www.delwp.vic.gov.au

May 2017

Regulatory impact statement Building Regulations 2017 Part A: Background and proposed approach

Acknowledgements

The Department of Environment, Land, Water and Planning acknowledges its responsibility for this regulatory impact statement (RIS) that sets out the proposed Building Regulations 2017 and the supporting argument for those Regulations.

This RIS has been prepared for the express purpose of supporting the proposed Building Regulations 2017 and other potential uses of the information contained in the RIS have not been a consideration in its development. The information contained in this RIS has not been subjected to an audit or any other form of independent verification.

© The State of Victoria Department of Environment, Land, Water and Planning 2017

This work is licensed under a Creative Commons Attribution 4.0 International licence. You are free to re-use the work under that licence on the condition that you credit the State of Victoria as author. The licence does not apply to any images, photographs or branding, including the Victorian Coat of Arms, the Victorian Government logo and the Department of Environment, Land, Water and Planning logo. To view a copy of this licence, visit http://creativecommons.org/licenses/by/4.0/.

ISBN 978-1-76047-592-5 (online)

Disclaimer

This publication may be of assistance to you but the State of Victoria and its employees do not guarantee that the publication is without flaw of any kind or is wholly appropriate for your particular purposes and therefore disclaims all liability for any error, loss or other consequence which may arise from you relying on any information in this publication.

Accessibility

If you would like to receive this publication in an alternative format, please telephone the department's Customer Service Centre on 136186, email customer.service@delwp.vic.gov.au or contact us via the National Relay Service on 133 677 or at www.relayservice.com.au. This document is also available on the internet at www.delwp.vic.gov.au.

# Contents

Contents	1

Acronyms and abbreviations	3

1	About building in Victoria	4

1.1	The Building Act 1993	4

1.1.1	About the Act	4

1.1.2	Building regulatory system under the Act	4

1.1.3	Building permit system	7

1.2	Responsibility for implementing the Regulations	9

1.2.1	Victorian Building Authority role	9

1.2.2	Council role	11

1.2.3	Building surveyor role	11

1.2.4	Other major legislative frameworks that interact with building controls	12

1.3	Rationale for government intervention	17

1.3.1	Imperfect or asymmetric information	17

1.3.2	Externalities	19

1.3.3	Other rationales for intervention	19

1.4	Evidence of building industry problems	20

1.4.1	Defective building work	20

1.4.2	Disputes	21

1.4.3	Complaints and investigations	23

2	Objectives and areas of focus	25

2.1	Objectives of the proposed Regulations	25

2.2	RIS areas of focus	25

2.2.1	Building reform program	26

2.2.2	Stakeholder input	28

2.2.3	Risk and impact assessment	29

2.3	Choice of options	31

3	Assessment of the proposed Regulations	33

3.1	Summary of preferred options	33

3.1.1	Design	33

3.1.2	Construction	34

3.1.3	Completion / use	35

3.1.4	Building practitioner registration	36

3.2	Summary of proposed Regulations	37

3.3	Summary of cost–benefit analyses	41

3.4	Competition assessment	43

3.5	Small business impacts	45

3.6	Evidence improvement project	46

3.7	Regulatory fees review	47

4	Evaluation	49

4.1	Evidence improvement project	50

4.1.1	Key performance indicators and baseline data	51

4.1.2	Evidence collection to support the midterm evaluation	51

4.1.3	Evaluation strategy program	52

5	Regulation change summary	54

Pt 1—Preliminary	55

Pt 2—Building Code of Australia	57

Pt 3—Owner builders	58

Pt 4—Combined allotments	60

Pt 5—Building permits	62

Pt 6—Siting	72

Pt 7—Projections beyond street alignment	78

Pt 8—Protection of adjoining property and the public	80

Pt 9—Building work	83

Pt 10—Building work—safety requirements	86

Pt 11—Designation of special areas for building work	90

Pt 12—Bushfire safety	92

Pt 13—Inspections notices and orders	95

Pt 14—Occupancy Permits and Certificates of Final Inspection	97

Pt 15—Places of public entertainment	104

Pt 16—Maintenance of buildings and places of public entertainment	105

Pt 17—Existing buildings	109

Pt 18—Building product accreditation	112

Pt 19—Building practitioners	116

Pt 20—Appeals and reviews	121

Pt 21—Infringement notices	122

Pt 22—Exemptions	123

Pt 23—Transitional provisions	125

# Acronyms and abbreviations

| Acronym | Name |
| --- | --- |
| Act, the | Building Act 1993 |
| BAB | Building Appeals Board |
| BAL | bushfire attack level |
| BRAC | Building Regulations Advisory Committee |
| CAV | Consumer Affairs Victoria |
| CFA | Country Fire Authority |
| current Regulations | Building Regulations 2006 |
| Department, the | Department of Environment, Land, Water and Planning |
| ESM | Essential safety measures |
| MBS | Municipal building surveyor |
| MFB | Metropolitan Fire Brigade |
| NCC | National Construction Code |
| NPV | Net present value |
| OHS | Occupational health and safety |
| PBS | Private building surveyor |
| PCA | Plumbing Code of Australia |
| PoPE | Place of public entertainment |
| proposed Regulations | Building Regulations 2017 |
| RBS | Relevant building surveyor |
| RIS | Regulatory impact statement |
| VBA | Victorian Building Authority |
| VCAT | Victorian Civil & Administrative Tribunal |

#### Abbreviations for Acts and Regulations

| Abbreviation | Name |
| --- | --- |
| Clause(s) | Cl(s) |
| Division(s) | Div(s) |
| Part(s) | Pt(s) |
| Regulation(s) | Reg(s) |
| Schedule(s) | Sch(s) |
| Section(s) | S(s) |
| Subdivision(s) | Sub-div(s) |

# 1	About building in Victoria

## 1.1	The Building Act 1993

### 1.1.1	About the Act

Before 1994, the legal framework for regulating building construction, standards and maintenance in Victoria was outlined in the Building Control Act 1981. Under this framework, the building approval system was administered by councils and only municipal building surveyors (MBSs) had the authority to issue building permits.

In the early 1990s, the Building Control Act 1981 was reviewed and the National Model Building Act was used as the basis for its replacement. Changes were subsequently reflected in the Building Bill that was passed by the Victorian Parliament in December 1993, giving rise to the Building Act 1993 (the Act).

The Act introduced major changes in building control to improve Victoria's building industry. These included:

removing MBSs' monopoly on issuing building permits, thus allowing private building surveyors (PBSs) to enter the market

introducing compulsory registration and insurance for builders and certain other categories of building practitioners, a system overseen by the Building Practitioners Board

creating the Building Control Commission, funded by a statutory levy

creating a statutory registration body, the Building Practitioners Board, and an appeal body, the Building Appeals Board (BAB).

The Act introduced building permit approvals by PBSs. As highlighted by the second reading speech for the Building Bill 1993, the objective of allowing PBSs into the market was to increase the efficiency of the approval system and resolve issues over council delays in issuing building permits. The ability to issue building permits through either a MBS or PBS was viewed as a means of generating competition in the industry.

### 1.1.2	Building regulatory system under the Act

The Act and building Regulations regulate building work, which is work for or in connection with the construction, demolition or removal of a building. The objectives of the Act are to protect the safety and health of people who use buildings and places of public entertainment (PoPEs), to enhance the amenity of buildings, to facilitate the adoption of national building standards, to facilitate the cost-effective construction and maintenance of buildings, to facilitate the construction of environmentally and energy-efficient buildings and to aid in the achievement of an efficient and competitive building industry.

The building industry is currently regulated through the following legislation and subordinate legislation.

The Act sets out the legal framework for regulating building work, building standards, building practitioners, the system for issuing building permits and occupancy permits, and the administration and enforcement of related building and safety matters.

Authorised by the Act, the building Regulations 2006 (the current Regulations):

prescribe standards for the construction and demolition of buildings and standards for the safety of PoPEs

regulate matters relating to the use and maintenance of buildings and PoPEs

set out the requirements for building permits, inspections and occupancy permits

provide for the accreditation of building products and construction methods

provide for the registration of building practitioners

prescribe fees and other matters for the purposes of the Act.

The National Construction Code (NCC) is called up and defined within the Act. The NCC is a set of technical provisions that incorporate all onsite construction requirements. Volumes One and Two of the NCC include the Building Code of Australia (BCA) and Volume Three includes the Plumbing Code of Australia (PCA). These were stand­alone codes before their consolidation into the NCC.

The BCA serves as a technical reference for the provisions around the design and construction of buildings and other structures, while the PCA regulates aspects of plumbing and draining installations. The NCC is performance-based and thus defines the outcome to be achieved without prescribing a particular method. It provides scope for states and territories to implement additional requirements (variations). S 109 of the current Regulations incorporate the BCA into the current Regulations.

Error! Reference source not found. is a conceptual outline of Victoria's regulatory system.

Building Act 1993

current Regulations

Building Code of Australia and referenced standards

Occupancy permits

Supporting building regulatory activity

Dispute resolution & enforcement

Regulation of building activity

Design phase

Construction phase

Completion and use phase

Building permit

Registration of building practitioners (incl. building surveyors)

Accreditation of products and processes

Figure 1: Conceptual outline of the building regulatory system

Source: Adapted from: Australian Government Productivity Commission, (2004), 'Reform of Building Regulation', p.xxvi.

Building surveyors are responsible for delivering most core regulatory activities including the issuing of planning / building permits during the design phase, the carrying-out of inspections during the construction phase and the issuing of occupancy permits at the completion phase. Supporting regulatory activities include:

practitioner licensing (by the Victorian Building Authority [VBA])

accreditation of products and processes (by the Building Regulations Advisory Committee [BRAC])

monitoring and enforcement (by VBA, local governments, Energy Safe Victoria and the chief officers of the Country Fire Authority [CFA] and Metropolitan Fire Brigade [MFB])

insurance

dispute resolution.

The NCC is updated on a three-year cycle and each update can introduce new design and construction standards. Over time, existing buildings may become noncompliant with current standards under the NCC, as well as any new standards introduced by the Act or Regulations. Generally speaking, the building regulatory system does not require existing buildings to be upgraded to current standards unless the building is being substantially changed or in exceptional cases where the building poses a risk to life or safety.

### 1.1.3	Building permit system

The Act sets out the process for making an application for a building permit to a MBS or PBS – the relevant building surveyor (RBS) – and also the process the RBS must follow to decide an application. As a result, it is the Act that imposes the substantive compliance burden on the owner to make an application for a building permit. It is also the Act that imposes the substantive compliance burden on the RBS to make a decision once that application is received by issuing or refusing a building permit.

The building permit enables registered building surveyors with statutory functions, powers and duties to check, verify and require the compliance of individual building projects with current building standards. The RBS must not issue a building permit unless satisfied that, among other things, the proposed building work complies with the relevant legislation, Regulations and the NCC. The Act requires the RBS to:

be satisfied that the documentation prepared for the proposed building work is sufficient

inspect building work during the construction phase including at the foundation, reinforcement, frame and prior-to-occupancy stages

issue directions to fix building work that does not comply with the Act, Regulations or the building permit issued in relation to the building work

on the completion of building work, issue an occupancy permit if the building is suitable for occupation or a certificate of final inspection if an occupancy permit is not required by the building permit.

Building permits are granted for a specified time period within which building work must be commenced and completed.

A broad outline of the building permit process is provided in the Victorian Competition and Efficiency Commission's Housing regulation in Victoria: building better outcomes report, as shown in Figure 2.

Submit all required documents including plans, specifications, soil reports, certificate of title

Pay fees and levy

Building surveyor

Assess application

Applicant

Apply for a building permit

Choose a building surveyor

Municipal building surveyor

Private building surveyor

Send notification to council

Commence construction

Issue building permit

Conduct mandatory inspections

Issue occupancy permit or certificate of final inspection

Figure 2: Building permit process

Source: Victorian Competition and Efficiency Commission 2005, Housing regulation in Victoria: building better outcomes, p 140.

In 2015–16 there were:

110,069 building permits reported in Victoria (an increase of about 4.5% on the previous year)

$31.5 billion building permit cost of works (an increase of 12.5% on the previous year)

48,872 registered and/or licensed building and plumbing practitioners.

## 1.2	Responsibility for implementing the Regulations

VBA, as the regulator, plays a major part in enforcing the Regulations. The other major players in regulating building in Victoria are councils, Energy Safe Victoria, the chief officers of MFB and CFA and building surveyors.

Other regulatory mechanisms that affect the industry are:

Planning and Environment Act 1987

Major Domestic Building Contracts Act 1995

domestic building insurance provisions

consumer protection arrangements

public health and safety controls.

### 1.2.1	Victorian Building Authority role

VBA is a statutory authority that oversees building control in Victoria. VBA was established on 1 July 2013 and assumed the functions of the Building Commission and the Plumbing Industry Commission.

The building-related functions of the VBA set out in s 197 of the Act are to:

monitor and enforce compliance with the provisions of the Act and Regulations

administer the scheme under pt. 11 for the registration of building practitioners

supervise and monitor the conduct and ability to practise of registered building practitioners

make recommendations to the Minister about the qualifications for registration of building practitioners

issue certificates of consent to owner-builders

participate on behalf of Victoria in the development of national building standards

monitor developments relevant to the regulation of building standards in Victoria

provide information (including to consumers) about matters relating to building standards, the regulation of buildings, and building work and building practitioners

provide information and training to assist persons and bodies in carrying out functions under the Act and Regulations

promote the resolution of consumer complaints about work carried out by builders

conduct or promote research relating to the regulation of the building industry in Victoria and to report on the outcomes of this research in its report of operations under the Financial Management Act 1994

monitor the system of collection of the building permit levy

charge and collect fees (determined in accordance with the Act and Regulations) for information and training services provided by VBA

administer the Victorian Building Authority Fund

advise the Minister on the carrying-out of VBA’s functions, powers and duties under the Act and on any other matter referred to it by the Minister

generally, carry out any other function or duty conferred on it by or under the Act or any other Act or under any agreement to which the state is party.

The Act currently establishes three statutory bodies.

Building Advisory Council – This is a senior, industry-based advisory group that advises the Minister for Planning about the administration of the Act (other than pt. 12A) and Regulations, and the impact on the building regulations system by Regulations made under any other Act, issues relating to the building permit levy, and any matter referred to it by the Minister.

Building Appeals Board – This is an independent statutory body that hears and determines appeals and disputes in relation to building control matters. It can waive, modify or vary the provisions of particular Regulations in particular cases. It can also determine that a particular design or element of a building complies with the Act and Regulations.

BAB provides services under three categories:

disputes about the applicability of the Regulations, adjoining property work and inspections

reviews of refusal of building and occupancy permits, report and consent of the Regulations and building notices and/or building orders

referrals and modifications regarding whether a design or element of a building complies with the Act or Regulations, or whether the provisions of the Regulations do not apply to a particular building.

Building Regulations Advisory Committee – This is a panel of building industry representatives that advise the Minister for Planning about draft building Regulations; accredits building products, construction methods and components or systems connected with building work; and advise VBA on technical, administrative and regulatory issues.

VBA’s functions are funded by owners through a levy on building permits, registration fees paid by practitioners and fines and costs levied through the practitioner discipline system. In 2015–16, the total building permit levy collected was $30.3 million, practitioner registrations and licensing accounted for $8.3 million, and VBA’s total annual income was $53.8 million.

### 1.2.2	Council role

A council is responsible for the administration and enforcement of the following parts of the Act within its local government area:

pt. 3 – Building permits

pt. 4 – Inspection of building work

pt. 5 – Occupation of buildings and PoPEs

pt. 6 – Protection of adjoining property

pt. 8 – Enforcement of safety and building standards.

In addition to other relevant statutory authorities, councils also have jurisdiction under planning, public health and local laws for matters relating to the use of buildings.

### 1.2.3	Building surveyor role

A surveyors can be either municipal or private.

A MBS is employed or engaged by a council and has additional duties of enforcing laws within its local government area in order to maintain community health and safety.

A PBS is a private person entitled to discharge functions, powers and duties under the Act following appointment in relation to building work. A PBS has additional reporting requirements and must notify the council of an appointment in relation to building work within its local government area. A PBS may specialise in certain areas (such as disability access or fire safety) or in relation to a particular type of building.

Where a MBS is engaged or a PBS is appointed in relation to a specific building project, they are referred to in the Act as the RBS.

The RBS carries:

issues building permits

carries out inspections of buildings and building work

issues occupancy permits and temporary approvals.

Specifically, a RBS's responsibilities include:

assessing and approving permit applications

certifying plans and structures in accordance with legislation

ensuring the consent of a reporting authority is obtained when required by the Act or Regulations

lodging prescribed documents with the relevant council

inspecting building work

verifying building works have been carried out in accordance with the building permit

issuing directions to fix building work

serving compliance directions, notices and orders where necessary

issuing occupancy permits and certificates of final inspection.

A RBS's responsibilities apply to a broad range of buildings and structures including:

commercial buildings (office buildings, factories and aged care facilities)

retail buildings (shops and shopping centres)

public buildings (hospitals, prisons, schools, parking complexes and marinas)

residential properties (houses, apartments, masts and antennae and swimming pools).

There are several requirements to become a registered building surveyor. In addition to tertiary qualifications in building surveying, surveyors must also have at least four years of practical experience. Once registered, building surveyors must have professional liability insurance as specified in the Building Practitioners' Insurance Ministerial Order under s 135 of the Act. Part C of this regulatory impact statement (RIS) has further explanation of registration requirements.

As mentioned previously, the RBS must cause building work to be inspected during the construction phase, specifically at the foundation, reinforcement, frame and before-occupancy stages at a minimum.

### 1.2.4	Other major legislative frameworks that interact with building controls

The building system alone is not capable of providing for all desired outcomes and the legislation works by an integrated approach with planning, health and consumer law.

Victoria's building regulatory system is governed by a number of laws and Regulations which do not fall within the scope of the this review of sunsetting Regulations.

#### Planning and Environment Act 1987

The use and development of land is controlled by the Planning and Environment Act 1987. A proposed use or development of land may trigger a requirement to obtain a planning permit under the Planning and Environment Act, depending on the requirements of the relevant planning scheme. A building permit is required to carry out building work on land under the Act, unless that building work is exempt from the requirement to obtain a building permit.

A planning permit allows land to be subdivided, developed or used for a particular purpose. Planning permits are issued by a council in accordance with their planning scheme under the Planning and Environment Act. Building permits relate to the construction process and are issued by the RBS under the Regulations.

If a planning permit is required, it must be issued before the issue of a building permit. A building permit must be consistent with requirements of a planning permit.

Table 1 sets out the key areas that are subject to planning controls and how those controls operate.

Table 1: Planning and building responses to address natural and human-made hazards and access barriers

| Key areas | Planning response | Building response |
| --- | --- | --- |
| Siting (privacy, sight lines and access to sunlight) | Victoria Planning Provisions | Pt 4 of the Regulations replicate the siting provisions of the planning framework |
| Bushfire management | Overlays Victoria Planning Provisions | BCA building standards for bushfire-prone areas Firefighting services Report and consent Mapping |
| Flood areas | Zoning Overlays | BCA building standards Report and consent Mapping |
| Salinity management | Overlay | Not regulated |
| Alpine areas Termite-risk areas | Victoria Planning Provisions | BCA building standards Mapping |
| Noise (industry, live music, urban, roads, mining and wind industry) | Noise attenuation Buffers Zoning Provisions | BCA building standards for noise attenuation to the internal perimeter walls of apartment dwellings |
| Accessibility for people with a disability | Victoria Planning Provisions for access into residential buildings | BCA building standards for nonresidential buildings based on the (C'th) disability standards |
| Licensed premises | Victoria Planning Provisions | BCA building standards |
| Odour / air quality (feed lots, broiler industry, tips / waste, dust / asbestos) | Buffers Zoning Overlays | Occupational health and safety (OHS) requirements |

A proportion of building work will require a planning permit. Decisions about planning permits should be consistent with the purpose and objectives of the Planning and Environment Act relating to the use, development and protection of land. For example, achieving planning framework environmental and sustainability objectives in relation to buildings could be achieved by siting policies or provisions at the urban and allotment scale. Siting provisions could reduce energy use or stormwater run-off from an allotment and enable accessibility to a building.

In contrast, amenity objectives in the building framework and decisions about building permits relate to the building shell and facilities contained in the building including that a structure facilitates an energy-efficient building, internal access to light and ventilation, sanitary facilities and access for people with a disability.

#### Consumer protection

The Domestic Building Contracts Act 1995 sets out the framework for regulating domestic building contracts and resolving disputes. It also states that a builder must not enter into a major domestic building contract unless the contract sets out details of the required insurance under the Building Act that applies to the work to be carried out under the contract.

Consumer Affairs Victoria (CAV) has administrative responsibility for the Domestic Building Contracts Act, the objects of which are to:

provide for the maintenance of proper standards in the carrying-out of domestic building work in a way that is fair to both builders and building owners

enable disputes involving domestic building work to be resolved as quickly, efficiently and cheaply as possible, having regard to the need for fairness

enable building owners to have access to insurance funds if building work under a major domestic building contract is incomplete or defective.

Remedies which may be available for poor conduct or service in the delivery of consultancy or construction services are:

a person can refer a matter about the conduct or ability to practise of a registered building practitioner to VBA for an inquiry

a person will be able to engage in conciliation or to apply for a dispute resolution order from the chief dispute resolution officer once amendments to the Domestic Building Contracts Act commence

a person may claim repair, refund or compensation under Australian Consumer Law administered by the Australian Competition and Consumer Commission.

The Domestic Building Contracts Act will continue to provide for CAV's conciliation services in relation to domestic building disputes until amendments made by the Building Legislation Amendment (Consumer Protection) Act 2016 commence. These must commence on or before 1 July 2017. Until then, CAV may only accept complaints or disputes for conciliation which are reasonably likely to be resolved. Under pt. 3A of the Domestic Building Contracts Act, participation in conciliation is voluntary and CAV cannot enforce conciliation outcomes. While the Domestic Building Contracts Act aims for timely, low-cost dispute resolution, in practice some building disputes are either not accepted for conciliation or not satisfactorily resolved through the conciliation process. In these cases, the consumer or building practitioner's only remaining option is to pursue the matter through the Victorian Civil & Administrative Tribunal (VCAT) or the courts.

The Building Legislation Amendment (Consumer Protection) Act 2016 will replace the existing conciliation provisions in pt. 3A and pt. 4 of the Domestic Building Contracts Act not later than 1 July 2017.

The amendments will create a new process for the conciliation and resolution of disputes to be delivered by a new service to be known as Domestic Building Dispute Resolution Victoria. The new service will be headed by a person titled Chief Dispute Resolution Officer who will be supported by suitably qualified conciliators and technical assessors capable of examining domestic building work. This new service will be funded from the existing Domestic Builders Fund, which is administered by CAV.

Conciliation at Domestic Building Dispute Resolution Victoria will be mandatory before an application can be made to VCAT or a court. Save for injunctive relief, participation in conciliation or the issue of a certificate stating that the dispute was not suitable for conciliation or could not be conciliated will be required before a party can access the more-costly dispute resolution procedures associated with a full VCAT hearing.

Where the parties cannot reach agreement through conciliation, the Chief Dispute Resolution Officer may issue a dispute resolution order. A dispute resolution order may require rectification of defective or incomplete work, or payment to a builder, or payment into a trust fund pending completion of rectification work.

In the most extreme cases, a dispute resolution order will also be able to be used to compel a builder to meet the cost of rectification by an alternative builder where the building work is so poor it would not be reasonable to allow the original builder to attempt rectification.

While a party who is required to comply with a dispute resolution order will have a right to seek review of the order at VCAT, there will be strong incentives to both parties to accept the outcomes of conciliation and the dispute resolution order, if one has been issued. VCAT will have the power to make costs orders if it finds the review application frivolous or lacking in substance, or if the party seeking review does not get a better outcome compared to that achieved through conciliation and the dispute resolution order. As a further incentive, breach of a dispute resolution order by a builder will also be a ground for disciplinary action.

CAV also aims to assist Victorian builders, owner-builders and consumers to avoid home building disputes by providing information about domestic building contracts and through its compliance activities, including the Better Business Initiative. It aims to assist consumers and builders to work together to solve building disputes. Its services include advice and conciliation.

Additionally, VCAT, established under the Victorian Civil and Administrative Tribunal Act 1998, has two main areas of involvement:

a domestic building list for handling disputes between home owners, builders, subcontractors, architects, engineers and other building practitioners

hearing appeals of VBA’s practitioner registration and disciplinary decisions.

Regulations also impose costs on councils, MBSs and fire authorities to undertake compliance and enforcement activities. This will largely be related to undertaking inspections and serving infringement notices where breaches are detected.

#### Domestic building insurance

Builders in Victoria are required by law to take out domestic building insurance, also known as builder's warranty insurance, for work valued at more than $16,000. The Victorian Managed Insurance Authority has offered domestic building insurance in Victoria at the direction of the state government since May 2010. Insurance provides cover to homeowners for incomplete or defective building work.

#### Public health and safety

There are public health controls which link to the use of a building. For example, a Class 1b or Class 3 building that is a rooming house must be registered on the public register of rooming houses maintained by CAV and comply with standards and requirements for prescribed accommodation under the Public Health and Wellbeing Regulations 2009.

The interface between building controls and health controls are set out in Table 2.

Table 2: Building and health responses to address public health issues

| Key areas | Building response | Health response |
| --- | --- | --- |
| Prescribed accommodation | BCA building standards for classes of buildings | Public health standards for facilities (resident numbers, room sizes, sanitary facilities, maintenance of facilities) |
| Rooming houses (building and fit-out) | - BCA building standards for classes of buildings | Registration Residential tenancy standards for fittings (locks, power, blinds, kitchen, lighting, security and maintenance of gas and electrical installations) |
| Hazardous materials (asbestos) |  | OHS-prohibited material Australian border protection State safety inspectors |

Decisions about the establishment and operation of a construction site must be informed the requirements of the Occupational Health and Safety Act 2004 and Occupational Health and Safety Regulations 2007 – see Table 3.

Table 3: OHS, health and local law responses to address construction site safety, health and amenity

| Key areas | OHS response | Health response | Local laws response |
| --- | --- | --- | --- |
| Safety Health Amenity | Head contractor to implement a health and safety coordination plan for site activity Employers to prepare a safe work method statement for high-risk construction work | Asbestos a prohibited material which must not be manufactured, supplied, sold or fixed or installed in a building Guidelines for nonresidential construction noise Offence to emit construction noise where it can be heard in residential premises | To protect adjoining land and assets, and the health and safety of people who work on the site and the public Asset protection permit Bond for protecting public infrastructure (such as roads and nature strips) Local laws (including to apply environment protection Regulations) |

Councils can assist with concerns about asbestos and often enforce noise laws under permits and local laws as part of their administration of local laws. WorkSafe Victoria is responsible for OHS compliance and onsite handling and transport of asbestos.

## 1.3	Rationale for government intervention

Many of the harms associated with the building industry reflect classic problems with competitive markets in which consumers are poorly placed to make well-informed decisions. Imperfect or asymmetric information and economic externalities are evident in the operations of the building sector and are discussed in detail in this section. Issues specifically related to each phase of building work (design, construction and completion / use) are explained in RIS Part B and issues related to building practitioners are explained in RIS Part C.

Harms that may potentially concern the building and construction process can be:

economic (such as property damage or financial loss)

health and safety (such as injury or loss of life)

environmental (such as pollution during or after construction)

social (for example, groups excluded from facilities or buildings)

cultural (for example, relating to the amenity of the streetscape)

aesthetic.

These harms may arise because of market failures in the building industry.

A range of nonregulatory solutions (or other legislative frameworks) may reduce the extent of these harms. These nonregulatory solutions and overlapping legislative frameworks must be considered when identifying the residual problem to be addressed. In the building context, these responses might include:

consumers seeking expert advice (for example, ArchiCentre home inspections)

general consumer protection laws and warranties

reputation effects and word-of-mouth

branding

professional associations (to provide information about practitioners' quality)

the resale market, where any short-term cost-cutting that is evident in poor workmanship and lesser-quality building products is likely to be reflected in lower selling prices, although cost-cutting with health and safety implications may not be as evident as poor workmanship or lesser-quality building products.

### 1.3.1	Imperfect or asymmetric information

Building occupants, home owners and end-users of public buildings typically represent a vast population that can potentially suffer from imperfect or asymmetric information with regard to the buildings they occupy. Asymmetric information can lead to problems with safety and quality which are addressed through the Regulations. Although complaints and defects may arise from quality issues, the primary objectives of the Act relate to safety, health and amenity. Quality is a matter that is primarily related to a contractual transaction between a consumer and a builder.

Builders are generally more informed than consumers about the features, quality and cost of building construction. For example, most consumers find it difficult to ascertain whether builders are providing the desired quality of service at a price that reflects the cost of delivering that service.

Consumers are likely to have less information than suppliers about key aspects of housing construction including structural soundness, the effectiveness of protection against fire and the use and impact of materials that could damage the health of occupants.

The purpose of actions targeting information asymmetries is to reduce transaction costs that are associated with acquiring the information needed to make an informed decision. Various reasons have been suggested as to why information asymmetries may be significant and/or persist within the building context. These include:

the long-lived nature of building assets means flaws may only become apparent in buildings several years after construction is completed

consumers are mostly infrequent purchasers of housing construction services, and this lack of experience makes it more difficult for them to assess and monitor the quality and cost of building work

consumers may find it difficult to collect information about whether the areas they intend to build or purchase are subject to any hazard (such as bushfire, flood or termites)

expenditure to construct a building is relatively high compared to most consumer purchases, so consumers may be more inclined to purchase services that are lower-cost, especially where this would substantially lower the overall cost, without understanding fully the trade-off with lower quality or safety

like many production processes, building construction is complex and involves a wide variety of skills and inputs, which can make it harder for consumers to assess the quality and cost of building construction services because they are unable to compare the quality and costs of inputs that create the final product being purchased

decisions about features of a building including the type of insulation, the orientation of the building and the fittings may be based on upfront cost, which can be more influenced by the builder who may have less regard to lifetime operating costs.

According to the Victorian Competition and Efficiency Commission, market institutions and strategies have evolved to help bridge the information disadvantage and reduce the resulting risks for consumers.

Private agents (such as architects and building surveyors) can represent the interests of consumers by negotiating with builders and/or monitoring their activities. But if many of the features of a building are difficult for anyone other than the builder to assess, then private agents may not be able to adequately assess some aspects of the construction process.

Associations representing suppliers of housing construction services also have a strong incentive to expand housing markets (for example, by identifying good-quality builders and reducing risks facing consumers). While the Housing Institute of Australia and the Master Builders Association focus on providing services to members, both also provide information and advice to consumers.

Many companies place a high commercial value on their reputations and market their services using testimonials from customers. Also, many consumers rely on the experience of family, friends and others in selecting suppliers. The housing construction sector, however, is considered to be different from many other markets because it comprises a large number of small-scale producers. This difference may make it harder or more costly for consumers to obtain information about the reputation of suppliers, compared with consumers in other markets. Also, given the time that may elapse before some quality problems become evident, short-term reputation may be an imperfect indicator for consumers.

To support their reputations, many companies voluntarily provide warranties for the services they provide. Warranties may help to reduce the risks facing consumers, but many consumers may place little value on voluntary warranties. A warranty may have limited value if, for example, it is particularly difficult for the consumer to show that housing defects stem from faulty work, or if suppliers faced with large claims can avoid responsibility by exiting the industry.

Consumers can use a variety of dispute resolution mechanisms to seek redress for poor quality services.

These institutions and strategies can help address the information disadvantage facing consumers, but may not overcome it. The reason for this limitation is that each mechanism may impose a considerable cost on consumers. Hiring a building surveyor or architect to vet quotes from builders and closely supervise construction, for example, may be too expensive for many consumers. And many consumers may place little value on the reputations or voluntary warranties of suppliers of housing construction services.

### 1.3.2	Externalities

Also known as a spillover effect, an externality is a harm felt by a third party, who is not involved in a building transaction. Third parties have no direct control over how a building is constructed, the materials that are used or the fittings and fixtures that are installed. For example, defective building work or unsuitable materials used in the construction of a building may pose a significant risk to users or occupiers of the building, who were not a party to the building transaction.

The costs of using materials that provide building resilience to hazards (such as fires) or the costs of installing safety features may increase the costs associated with constructing a building. Consumers of building services (who will not be ongoing users of the building) and builders may be motivated to reduce the costs of a building. Developers and investors may be less-motivated to incur additional costs for safety features from which they believe they will not benefit. The extent to which this is a problem or can lead to harm is more significant where a safety-related feature is not readily observable postconstruction.

Consumers who are not the occupiers of a property or are simply a short-term investor (such as a developer) may be less-likely to invest in the supply of materials and fixtures that will provide benefits over the life of a building.

Users of a building who are at most risk of injury or death in the event of a hazard are often not party to the building transaction.

### 1.3.3	Other rationales for intervention

If there isn’t a market failure, then there may be other rationales for government intervention such as to improve social conditions or manage public risk.

The Victorian Guide to Regulation identifies other nonmarket criteria which may be an appropriate rationale for government intervention including addressing social welfare objectives to achieve equity goals, promoting public health and safety, achieving cultural objectives and preserving and protecting environmental or historical resources. Other social policy objectives include human rights, protecting the vulnerable and disadvantaged and relieving geographic and social isolation.

In the building context, types of goals that might be relevant to government intervention include:

social inclusion (for example, in relation to people with mobility issues or other disabilities who face difficulties accessing facilities and buildings)

crime prevention (for example, in relation to lighting in buildings)

public health and safety (for example, in relation to structures intended for use by the broader community such as rental accommodation and PoPEs).

## 1.4	Evidence of building industry problems

According to the Victorian Auditor-General, a significant number of Victorian consumers experience problems with domestic building. Research by CAV in 2009 estimated that 16% of consumers in the Victorian domestic building market experienced problems. This sector had the highest proportion of consumers reporting detriment across all sectors. The 2016 Australian Consumer Survey found at least 17% of consumers who had undertaken home building, renovations, repairs or maintenance in Victoria had experienced a problem. Problems were likely to be related to poor workmanship and delays in getting substandard work fixed. The average estimated cost to resolve a problem was $1,321.02.

Different bodies are responsible for handling different types of issues:

building work on adjoining property (BAB)

practitioner function- and role-related issues (VBA, BAB)

building work / specific issues such as defective building work, noncompliant building work or incomplete building work (VBA and Building Advice and Conciliation Victoria, which is to be replaced by Domestic Building Dispute Resolution Victoria), VCAT)

unregistered building practitioners (VBA, Magistrates Court).

The high ranking of building industry complaints above those of many other regulated industries suggests that continued regulation is warranted. Further, according to the 2016 Australian Consumer Survey, over the five years to 2016 the level of complaints dropped by only 1%.

### 1.4.1	Defective building work

Research shows that defective building work, particularly in residential buildings, continues to be an issue in Victoria and nationally. A defect is classified as a shortfall in performance rather than just any fault in construction. Therefore by definition a defect is avoidable and would be avoided if the work was completed at an expected level of quality.

Research conducted on all new housing constructed in Victoria over a 15-year period between 1982–1997 investigated the prevalence, type and cost of defects in residential construction. The research found about 12.5% of new residential properties reported defective work of some kind. The costs of defects were both the claim value (the amount paid to the plaintiff by order of a tribunal, calculated as the cost to rectify the defect) and the associated costs (costs incurred to win the claim value such as investigation and assessment to prove the defect).

The research found average defect costs were $4,245 (in 2006 terms or about $5,400 present value) per dwelling where a defect claim occurred. This represented about 4% of the contract value of a dwelling at the time. The study also found most buildings with defects had more than one type of problem, although the most significant individual causes of costs were water ingress and footings defects.

It was also found that the type of construction had an impact on the cost of defects. Generally, the more complex the construction, the higher the cost of any defects. The highest average total costs were for construction of an extension of an existing dwelling ($6,264 in 2006 terms or about $7,900 in present value) and subfloor improvements ($5,730 in 2006 terms, or about $7,200 in present value).

Preliminary analysis by the VBA of investigation case files suggests that at face value, a large majority of defects and resulting disputes relate to the quality of workmanship rather than safety. Analysis of defects that were found following inspections as part of Building Advice and Conciliation Victoria matters highlighted the following most prevalent items involving defects:

windows and doors (16% overall): the top five defects were workmanship - material damage, operation - poor operation, workmanship - poorly installed, seals - inadequate fixing and workmanship - incorrect gaps

plastering (9% overall): the top four defects were level of finish – damaged material, level of finish – incorrect level of finish, cracking – board/sheet and workmanship – poor finish, cracking – cornice

brickwork and masonry (8% overall): the top six defects were workmanship – poorly laid, workmanship – cleaning, workmanship – damaged materials, cracks – brick coach, specifications – brick and mortar – mortar colour.

### 1.4.2	Disputes

While there is likely to be some level of disputes in any industry where parties are subject to contractual obligations, the prevalence of disputes and disagreements between parties involved in a building project can be seen as evidence of particular problems associated with this industry. The nontechnical nature of consumers can be a cause of disputes, emphasising the effect that asymmetric information can have on the practicalities of a building project.

Resolving a dispute can involve direct costs (such as lawyer or other professional fees, application or inspection costs) and indirect costs (such as extra rent if completion is late or lost work due to the time required to resolve the issue).

The cost of building project disputes in Victoria was estimated to total $513 million a year in a 2005 study. About half the total – $258 million – was attributed to domestic building contracts and half – $255 million – to commercial contracts. The report expressed these costs in 2003–04 dollars so in present 2015–16 terms the costs of building project disputes calculated on the same basis (without adjustment for different levels of building activity) would be a total $691 million, with $348 million in the domestic sector and the lesser half in the commercial sector.

These costs estimates are detailed in Figure 3 by type of dispute and have been presented in 2015–16 values for currency and clarity in the current context. Figure 3 shows there are different levels of issues that can arise in a building project and the level of issue can impact the costs involved. For example, the most serious issues are defined as ‘dispute’ and ‘conflict’ and are considerably more costly because they introduce a third party to the issue, usually a lawyer or mediator. However, even a ‘concern’, which is a worry or anxiety about the project held by one party but not raised with the other, can still impose a cost.

Figure 3: Prevalence, type and cost of building project disputes

Source: Victoria Building Commission 2005, Dispute reduction research summary report, March 2005.

### 1.4.3	Complaints and investigations

VBA received a total of 3,887 building-related complaints from July 2010 to June 2015. 10% of these complaints were chosen randomly for further analysis, with roughly equal proportions from each year, resulting in a final sample of 370 files. This excludes any building order referrals.

Figure 4 shows the types of issues raised, with the most common relating to:

building work in adjoining properties (46% of complaints and including siting, resulting damage and protection work)

practitioner function- and role-related issues (29% of complaints and primarily relating to the RBS and builder)

building-work-specific issues (most commonly relating to defective, noncompliant and incomplete building work).

Specifically, the most commonly raised issues relate to:

siting (19% of complaints)

the function / role of the RBS (16% of complaints)

damage to adjoining properties (14% of complaints)

the function / role of the builder (12% of complaints).

Together, these four issues comprise over 60% of all complaints in the sample.

In relation to the building-work-specific issues in Figure 4, a distinction has been drawn between noncompliant building work and defective building work. In Figure 4, noncompliant building work is building work that contravenes the Act or Regulations and defective building work is building work that contravenes a building contract other than by way of a breach of the Act or Regulations.

Figure 4: Breakdown of complaints to VBA, July 2010 to June 2015

In addition to complaints made to VBA, there were 1,620 appeals or disputes received by BAB between 2009–13 of which:

46% related to siting issues

25% related to building work

16% related to protection work

11% related to decisions of the Building Practitioners Board (which was abolished and its functions transferred to VBA on 1 September 2016)

1% related to permits.

This indicates that siting complaints are common with respect to the Regulations, although it should be noted that siting disputes may arise in relation to the planning system as well as the Regulations. Building work issues (relating to defective, noncompliant or incomplete work) comprised a higher share of appeals and disputes to BAB compared to complaints made to VBA (25% compared to 14%).

# 2	Objectives and areas of focus

## 2.1	Objectives of the proposed Regulations

The Victorian Guide to Regulation requires that regulation is:

effective in addressing the underlying causes of harm

cost-effective

proportionate to the harm or risk to the community

flexible to accommodate changes in technology, markets, risks and community views

consistent with the government’s priorities to enhance Victoria’s liveability and growth in productivity and employment

consistent across the government to avoid unnecessary overlap and duplication

clear and easily to understand by business and the community.

The regulatory analysis supporting the RIS has examined the effectiveness of the current Regulations in the context of the broader building framework and objectives in the Act, which are to:

protect the safety and health of people who use buildings and PoPEs

enhance the amenity of buildings

facilitate the adoption and efficient application of national building standards;

facilitate the cost-effective construction and maintenance of building (and plumbing) systems

facilitate the construction of environmentally and energy-efficient buildings

aid the achievement of an efficient and competitive building (and plumbing) industry.

The building Regulations do not regulate matters relating to plumbing. These are dealt with in the Plumbing Regulations 2008.

## 2.2	RIS areas of focus

‘Sunsetting’ requires a Regulation to be remade after 10 years or it is automatically revoked. In Victoria, all Regulations are automatically revoked after 10 years in accordance with s 5 of the Subordinate Legislation Act 1994.

The current Regulations were due to sunset on 6 June 2016. In February 2016, the Subordinate Legislation (Building Regulations) Extension Regulations 2016 were made, extending the operation of the current Regulations until 5 June 2017. Interim Regulations are being prepared to extend these Regulations until 5 June 2018. However, the government intends to make replacement Regulations well ahead of the June 2018 expiry of the interim Regulations.

The rationale for sunsetting is that much regulation inevitably has a use-by date, when it is no longer needed or will require significant modification. Regulations may easily become out-of-date and create unnecessary burdens on citizens and businesses.

The objective of a mandated ten-year sunsetting provision is to ensure every Regulation is regularly reviewed in order to examine if it is still required and, if it is, how it might be improved to better achieve the desired benefits and/or reduce the cost it imposes.

A RIS for sunsetting Regulations should contain sufficient information to allow a decision to be made about whether the existing regulatory measure is still required, or should be modified, or whether other options are more appropriate. Given the scale and scope of the current Regulations, and that some the Regulations are more recent and reflect changes to the NCC, it was not considered appropriate to quantify each and every Regulation. A range of factors, illustrated in Figure 5, influenced the determination of what the key areas of focus should be in the RIS.

The Regulations are substantial in volume and complex and cover a range of topics relating to the design, construction / use and safety of buildings and PoPEs. The Regulations also provide detailed specifications relating to building permits, building inspections, occupancy permits, maintenance requirements and enforcement procedures. The Regulations also prescribe qualifications and other matters relating to the registration of building practitioners.

Figure 5: Factors considered in determining RIS focus areas

### 2.2.1	Building reform program

This review of the Regulations has occurred during a period of intense reform activity and in the context of government’s commitment to increase protection for Victorian consumers from building malpractice. The building reform program started with the development of legislation in 2015 to give effect to this commitment and to respond to the findings of the Victorian Auditor-General's performance audit that found the existing domestic building consumer protection framework does not adequately protect consumers.

The Building Legislation Amendment (Consumer Protection) Act 2016 passed through the parliament in April 2016 and includes amendments to:

restore confidence for consumers and building practitioners and underpin further growth in the state’s $2.8 billion building industry

achieve quality built outcomes

ensure timeliness in all processes

deliver clear and accessible pathways for consumers to resolve disputes and ensure their homes are built to the required standard.

In summary, the Building Legislation Amendment (Consumer Protection) Act includes provisions to:

improve dispute resolution, with a new Domestic Building Dispute Resolution Victoria Service, mandatory conciliation of disputes and enforceable dispute resolution orders

abolish the Building Practitioner Board and transfer its registration and disciplinary functions to VBA

improve registration standards including by introducing time-limited registrations with the requirement to demonstrate maintenance of competence on renewal of registration

improve oversight of building surveyors and building work

strengthen regulation of owner-builders

expand grounds for discipline

address the potential conflict of interest of building surveyors

improved and timely disciplinary process and new disciplinary santions.

Further reforms were introduced into parliament in December 2016 by the Building Amendment (Enforcement and Other Measures) Bill 2016. This Bill was passed by parliament on 11 May 2017 and will come into operation after it receives Royal Assent. This Bill continues the reforms to Victoria's building system that started with the Building Legislation Amendment (Consumer Protection) Act. It provides for:

making information for consumers better-quality and more available

reforms to practitioner registration including corporate registration, and for personal and financial probity requirements

increased flexibility in the regulation of building surveyors including facilitating transfers of building surveyors once appointed

publication of building practitioners' registration and disciplinary history to support informed consumer choice and promote confidence in the industry

reforms to the building permit system and building permit levy

reforms to make a builder named on a building permit responsible for compliance

new indictable offences

better regulation of building inspections to provide greater assurance to consumers that the person who carries out an inspection of building work is qualified to do so

stronger entry and information-gathering powers to enhance the ability of VBA and councils to be strong and effective regulators

clarification of the role of local government in administering and enforcing the Act

reformed building notice and building order provisions.

Further amendments will be made to the Regulations after the Building Amendment (Enforcement and Other Measures) Bill 2016 receives Royal Assent and comes into operation. These will further improve the system, and it is expected that as the reforms progress new or amended Regulations will be developed to operationalise the changes. Appropriate transition periods will be provided to allow for the orderly implementation of changes.

Regulations that appear to be affected by the reforms proposed by the Building Amendment (Enforcement and Other Measures) Bill will be remade in substantially the same form until after the Bill receives Royal Assent and comes into operation. This approach will have the benefit of avoiding any undue costs being incurred by practitioners and consumers in implementing regulatory change that may become redundant and will allow for a more streamlined implementation of reforms.

### 2.2.2	Stakeholder input

Stakeholder input and feedback was an important factor in understanding how the current regulatory system was working and determining key areas of focus for the RIS.

The department invited public submissions, with stakeholders asked to comment specifically on the Regulations they considered to be working well, on those not working well and on changes that could be considered.

From this process, a total of 374 submissions points were received on the current Regulations from 44 stakeholders. Submissions were received from a wide range of industry stakeholders including:

peak industry bodies

fire authorities

councils

individual practitioners.

Each submission was reviewed by department officers and a preliminary response to each submission was developed in consultation with technical advisers from the VBA.

The submission process confirmed that stakeholders (industry and regulators) support regulation and thought the current Regulations were generally working well. However, the assessment of submissions indicated there was scope to:

move to a more risk-based approach to regulating buildings and building work

reduce the complexity and inflexibility of regulatory requirements

streamline administrative processes

improve the design of the Regulations

recognise that nonregulatory options may be better at addressing harms and improving outcomes.

The policy areas that were the subject of a high volume of submissions related to Regulations assessed in RIS Part B:

in the design phase, building permits, report and consent and mandatory notifications for stages of building work

in the construction phase, qualifications and protection work

in the completion / use stage, essential safety measures (ESMs) and PoPEs.

The submission process also highlighted opportunities to reduce red tape, resulting in amendments having already been made to the current Regulations. These include aligning building permit timeframes for swimming pools, enabling building owners to maintain fire-related ESMs in accordance with the most current technical standard and enabling better alignment of siting and setback requirements across council planning schemes in relation to irregularly shaped blocks.

The department also reviewed ministerial correspondence received between 2007–11 to determine whether there were any particular issues associated with policy areas that were raised with the Minister via correspondence. In total, 673 pieces of correspondence were identified and a desktop analysis of each was undertaken to assess the quality of the evidence in relation to each issue.

This assessment identified the main issues raised in ministerial correspondence in relation to the current Regulations were:

in the design phase, report and consent, construction requirements associated with bushfires and stormwater drainage

in the construction phase, building work on adjoining property (particularly in relation to residential development and protection work), prescribed qualifications, energy efficiency and owner-builder requirements

in the completion / use phase, swimming pool barriers and disputes associated with defective building work.

### 2.2.3	Risk and impact assessment

The department undertook a preliminary review of the current Regulations to determine the need for them, whether any needed to be modified or new ones added and whether there are better alternative solutions. This was undertaken as a risk and impact assessment. Figure 6 summarises the methodology used for the assessment.

Preliminary

assessment



The regulations

in each policy

area were

reviewed to

determine if the

regulation was

still necessary

Data

collection

plans



Identification of

evidence

required and

possible

sources of

evidence to

enable the

regulations to

be reviewed in

accordance

with the Guide

Inspections

to Regulation

Stakeholder

submissions



Each

submission on

the policy area

was reviewed

Nature and

extent of the

problem



Collation and

analysis of

available

evidence to

determine the

nature and

extent of

problem and

the likelihood

of the problem

occurring

Options

analysis



Assessment of

options to

determine the

optimal

approach to

addressing the

cause of the

problem

Figure 6: Process flow of risk and impact assessment

To facilitate this process, the Regulations were divided into policy areas, to organise them into groups with a common underlying purpose or theme.

Each policy area was subject to preliminary assessment to identify what evidence would be required to review the Regulations in accordance with the Victorian Guide to Regulation. The evidence identified was then formatted into a data-collection plan and provided to VBA as a preliminary data request.

VBA then reviewed all the data-collection plans and categorised the requests into quantitative and qualitative data groups, identified the data owners, possible data sources and where necessary possible proxies that could be used in the absence of evidence. This stage of the process highlighted there were barriers to collecting and collating data to support the policy work.

Most data were held externally to the department. Data owners include VBA, CAV, municipal councils and PBSs. Not all data were held electronically by the relevant bodies and was often difficult to access, manipulate and analyse. Much evidence could only be collected by hard-file reviews or by reconciling different databases, both resource-intensive exercises. Ultimately, the best available option was building permit levy data.

Councils were identified as a source of significant qualitative and quantitative evidence. However, discussions with councils and reviews of available data indicated differences in the detail available and ease of its use and manipulation, making consistent comparisons across councils difficult. In addition, councils had limited resources available to support the data-collection process, particularly where data needed to be extracted, redacted and sorted. Much of the evidence made available by councils was consolidated, making any detailed analysis difficult (particularly comparing findings across councils). However, the data that were obtained have been used to the extent possible to support assessment of the current Regulations.

The first stage of the evidence-based policy analysis was identifying the nature and extent of the problem to be addressed. This is a threshold issue in the process of evaluating the need for government action or for continuing government intervention (for example, in the case of sunsetting Regulations).

The department identified the broad problem and market failure and then used the evidence available to understand the cause of a problem. Once this was identified, a risk assessment was conducted to determine the likelihood of the problem occurring and to quantify the impacts if the problem if it did occur.

Based on this analysis, if there was clear evidence of a problem, the department conducted an options analysis to determine the best approach to address the cause of the problem and determine if there was sufficient justification for government intervention through regulation.

This assessment was undertaken by considering the options available to address the problem, including an assessment of the market failures.

This process resulted in the development of policy position papers, which largely consisted of a desktop analysis, targeted consultation with relevant stakeholders and data that were able to be readily accessed, either from VBA or from councils. In addition, early stakeholder submissions also provided an insight into the need to retain, amend or to sunset current Regulations.

The department’s review of the current Regulations, which is set out in the policy position papers, has been collated into a summary table in Chapter A5 Regulatory change summary. Table 4 sets out the context of the Regulation and explains if it is still required and, if it is, how it might be improved to better achieve the desired benefits and/or reduce the cost it imposes.

Table 4: Key to summary table in Chapter A5 Regulatory change summary

| Revoke | No change | Update | Reduce | Strengthen |
| --- | --- | --- | --- | --- |
| The existing Regulation will be deleted. | No change to the Regulation is proposed. | An existing Regulation has been amended to clarify, improve, remove ambiguity and/or reinforce the policy intent of the Regulation. | An existing Regulation has been amended to reduce the burden (compliance, administrative and other costs). | An existing Regulation has been amended, or a new Regulation introduced, to respond to the problem. |

## 2.3	Choice of options

RIS parts B and C contain practical alternatives to achieve the desired objectives of the Regulations, in full or in part, and options to address any new issues arising in the building industry. These alternatives have been shaped in part by reference to building regulation in other jurisdictions, particularly less-onerous approaches.

Each of the options will have a range of impacts on stakeholders, which have been discussed in the cost–benefit analysis. These have been calculated as incremental costs or benefits on top of the base case.

The base case used for this analysis assumes that the existing Regulations are not remade nor are any new Regulations created.

Under this scenario, building industry participants would still be governed by the Act and other relevant legislation (such as the Occupational Health and Safety Act and the Planning and Environment Act) would still apply. The NCC would be the primary source of guidance to building industry participants but breaches of the NCC would not be enforceable under Victorian law because the NCC is adopted by the Regulations.

RIS parts B and C provide more detail as to how the base case would operate in relation to certain topics dealt with by the Regulations.

The analysis has been split into topic areas and options have been considered for each topic, which generally align with either of the following.

Option 1 – Remake the current Regulations: this option involves remaking the current Regulations without any changes. Without material changes to the current Regulations, it is not expected that any significant additional costs or benefits would be incurred by the building industry. This option provides costs and benefits that equate to the situation today.

Option 1.1 – Remake the existing Regulations with topic-specific changes: this option involves remaking the current Regulations with a variety of topic-specific changes, which are considered to be substantial. The topic-specific changes under each option are not mutually exclusive in all cases. Given broader reform and limited data availability to underpin the evidence base for change, options analysis is focused on topic-specific incremental changes to existing Regulations and addressing specific risks or reducing unnecessary regulatory obligations.

Alternatives considered in the RIS are confined to those which are within the scope of the Regulations being reviewed. That is, alternative options that require amendments to the Act or any other Acts or Regulations that impact on building regulation are outside the scope of the RIS and have not been considered.

More specifically, potential options that have not been considered in detail are listed below.

Industry guidelines: under this option, the Regulations would not be remade but instead industry guidelines would set out codes of best practice that would be likely to reflect most of the requirements in the Regulations. This alternative was considered but discounted because even with the Regulations there continues to be the problem of significant noncompliance with the Act and Regulations. It is reasonable to expect that noncompliance would increase if the Regulations were replaced with unenforceable guidelines.

Reliance on market mechanisms to overcome information asymmetry: reliance on market mechanisms would require analysis of the broader legislative framework, which is beyond the scope of the RIS. Therefore, this alternative has not been addressed. It is also very unlikely that market mechanisms would be sufficient to address the problems identified and remove the need for government intervention to assure public and private health and safety protection. However, it may be that market mechanisms are suitable for some areas or subgroups.

# 3	Assessment of the proposed Regulations

## 3.1	Summary of preferred options

The major changes proposed in the Regulations are set out below. RIS parts B and C explain these changes and other minor changes.

### 3.1.1	Design

The proposed changes to building Regulations covering the design phase are intended to give greater effect to recent amendments to the Act that make a building surveyor more accountable for assessment decisions when issuing a permit and lodging documents with a council.

Stakeholder feedback and evidence held by VBA indicate that a universal minimum requirement for documentation of building work could be excessive. Additional administrative burden would be placed on applicants and on building surveyors due to the need to provide and consider a standard set of information and documents, even when they are not required for a specific application to approve building work because of the nature of that particular building work. The proposed Regulations enable the RBS to make a professional judgement about what documents are required to assess a particular permit application.

Currently, there is considerable variance in the proportion of building permits submitted online. The current building Regulations are geared to manual lodgement of documents. Therefore, reducing the number of hard copies required will better support electronic lodgement and should result in time and cost efficiencies.

In order to ensure high levels of accountability, it is proposed to require building surveyors to provide more current and targeted information to VBA. The additional information relates to timeframes for building work and to specific obligations relevant to the work being carried out. This will include information related to mandatory inspections, exemptions and consents granted and whether the work relates to change of use or requirements for protection work.

Building inspectors will be required to provide information setting out time limits for building work, details of approved alternative solutions and BAB determinations. These will include the following.

A clear statement in the building permit form (current form 2, proposed form 4) that describes when a building permit lapses and the consequences of this. This amendment is designed to improve compliance by owners and building practitioners with regulatory timeframes for construction projects and to reduce the likelihood of a building permit lapsing inadvertently, which might give rise to the need to apply for a new building permit or building work continuing in the absence of a valid building permit and in breach of the Act.

More-specific details of performance solutions. These will be categorised in table form for ease of reference according to whether they relate to fire performance, bushfire performance or other matters. Details to be included relate to the method of approval, date of approval and details of applicable materials, systems, methods of building, procedures, specifications and other relevant requirements necessary to provide transparency and compliance in relation to the nature of the building solution that comprises the performance solution.

More-specific, targeted details of BAB determinations relevant to the building work (for example, in relation to protection works) where these are made before a building permit is issued. As with the details provided for alternative solutions, the inclusion of such details will provide transparency that such a determination has been made and also the nature of the determination (for example, by identifying whether any building Regulations have been varied in relation to the building work).

Projections can pose a risk to larger-vehicle traffic (such as trucks). In order to reduce the risks, the proposed Regulations require a uniform vertical clearance for a projection near pedestrian traffic (for example, over a footpath) and a second, higher, minimum acceptable vertical clearance for a projection proposed near vehicle traffic (for example, over a road). Where the Regulations provide a permitted clearance over the street alignment that is sufficiently long enough to mean that a truck or other larger vehicle could pass near or under the projection, the minimum height clearance for the projection is proposed to be raised from 2.7 m to 5 m. Amending to 5 m brings Victoria’s standard in line with what local governments in other jurisdictions allow for.

Other minor changes are proposed to facilitate the efficient administration of the building permit system through:

mandated application of forms for applying for a building permit that require disclosure of any relevant planning permits and proposals for ESMs

writing into Regulations the NCC requirement for a building surveyor to document the assessment of proposed performance solutions

requiring a builder to have relevant information onsite (rather than just taking reasonable steps to provide such information)

permitting the retention of documents by councils in electronic form rather than in their original forms

requiring building surveyors to forward a copy of a building permit to a builder and for builders to notify building surveyors of changes in their details

clarifying the wording related to setbacks

changing the way the average height of a wall is calculated to avoid the impact of the use of dwarf walls in calculating average height

clarifying fencing options for right of ways or laneways and farming allotments

improving the Regulations in relation to projections (such as by separating distinct design standards by street width and by providing for new and improved guidance material).

### 3.1.2	Construction

Regulations controlling the construction phase of a building are designed to ensure the satisfactory completion of building work and cover among other things time limits for building and mandatory inspections designed to ensure building work is both timely and protects the health and safety of the public and occupants.

Proposed changes to the building Regulations relating to the construction phase address issues related to incomplete building work, protection work on adjoining properties, mandatory notification stages and mandatory inspections of nondomestic building work.

In order to limit the inadvertent lapsing of a building permit that results in building work being undertaken illegally, the proposed Regulations require RBSs to send the owner and builder a notice of the imminent lapse of a building permit, in certain circumstances, three months before the permit is due to lapse. This practice is currently a best practice but voluntary step. The department estimates an additional 8,000 such notices will be issued annually. The notice will also need to be sent to VBA and the relevant council. The proposed Regulations also allow an applicant to apply in writing for an extension to prevent a permit lapsing.

Owners of property adjoining building works often have insufficient information about protection work, which can lead to delays in the building process. This problem can be addressed by providing additional information to what is required under the current Regulations, including:

VBA-approved guidance material that clearly explains the operation of the protection work process and the procedure for resolution of any dispute arising as a result of a breakdown in this process

a copy of an allotment plan showing the physical relationship of proposed building work to an adjoining property

plans and specifications showing how the proposed building work may impact on adjoining property and how the protection work proposed addresses this risk.

In order to address common building defects related to the construction of a new building, the alteration of an existing building, the demolition or removal of a building or the construction of a swimming pool, three new mandatory notification stages (increasing the number of such stages from four to seven) will address concerns raised by stakeholders and the high incidence of defects related to waterproofing, fire rating requirements and stormwater drainage systems. The three new mandatory notification stages are:

before covering walls, floors or ceilings, to check fire-resistance levels comply with the BCA and the structural integrity of the framework has been retained as a result of subsequent work (for example, electrical and plumbing work)

before covering waterproofing in wet areas

completion of any external drainage / stormwater drainage system.

As indicated above, the proposed changes to notification stages aim to address high levels of defects that are evident in contemporary building works. They are proposed even though information available to the department suggests that the decrease in the number of defects that would be needed to balance the additional cost of the Regulation is unlikely to be achieved.

Other minor amendments include:

increasing the bond value to $10,000

rewording Regulations related to specific matters a building surveyor must take into account when deciding if protection work is required and the relationship between the need for protection work and the provision of a s 238 certificate

requiring additional information in emergency orders.

### 3.1.3	Completion / use

Occupancy certificates or certificates of final inspection variously determine when a building can be occupied, how a building is used and when and how the use of a building may change. An occupancy certificate is necessary if required by a building permit, and a certificate of final inspection is issued where an occupancy certificate is not required. These certificates are only required for buildings or places that require a building permit.

The current requirements for PoPEs capture ‘places’ that may include a natural or fenced enclosure. Stakeholders have submitted that building surveyors are not well-placed to assess applications about gatherings and events held in open spaces (such as parks and gardens).

Research during the planning stage for the proposed Regulations found a lack of understanding by building owners of their maintenance obligations and a lack of compliance with annual reporting requirements. These shortcomings arise from complex Regulations and lack of clarity around annual reporting.

The proposed Regulations:

set uniform requirements for all buildings by removing historic differences for pre- and post-1984 buildings

establish a requirement for a stand-alone maintenance schedule (separate from conditions attached to an occupancy certificate) when an occupancy certificate is issued or amended or a maintenance determination has been issued

provide for approved forms and prescribed information requirements for annual reporting, in order to streamline reporting and reduce administrative burden

provide for approved forms for the stand-alone maintenance schedule to reduce administrative burden.

Further, stakeholder consultations indicate that, while reliable information does not exist as to the location, age, state and type of swimming pool and spa barriers in Victoria, swimming pool and spa barrier requirements are complex and confusing with three design standards for pools and spa barriers applying in Victoria, depending on when the pool or spa was constructed.

The proposed Regulations will require all pools and spas to erect a barrier that complies with the most recent Australian Standard, AS 1926.1-2007 (a four-sided isolation barrier). Existing barriers that do not comply with AS 1926.1-2007 would need to be replaced or substantially updated to comply with the requirements of the Australian Standard.

Other minor changes include:

clarifying obligations in the Act and Regulations which require additional information to be lodged with a council

improvements to current form 5 (Application for an occupancy permit), form 6 (Occupancy permit) and form 7 (Certificate of final inspection).

removing the Melbourne Cricket Ground as a prescribed PoPE under reg 1103(b) of the current Regulations to provide for consistency with the treatment of other sports grounds.

### 3.1.4	Building practitioner registration

The proposed Regulations aim to minimise the need for VBA to request additional information from applicants for registration by providing a form for applying for registration and stipulating the information that must be provided with the form including matters such as proof of identity, work or employment history and certified copies of certificates.

Further changes to Regulations related to building practitioner registration will come into effect on 1 July 2017 following the start of the balance of the Building Legislation Amendment (Enforcement and Other Measures) Act 2016.

Currently the fees and charges in relation to building practitioners are being remade, as they currently are in the current Regulations, without any changes. Once these changes have come into effect and VBA has undertaken a review of services, taking into account the efficiencies derived by establishing a modern information technology platform to manage business information requirements, consideration of fees and charges relating to the registration of building practitioners will be assessed. This review is part of the proposed work explained in Chapter A3.7 Regulatory fee review.

## 3.2	Summary of proposed Regulations

RIS Parts B and C demonstrate the department considers the proposed Regulations will have the highest net benefit of all the options examined. While data are lacking in many cases, this conclusion reflects various judgements and assumptions made by the department.

This section summarises the proposed Regulations.

##### Pt 1 – Preliminary

Pt 1 sets out the objectives of the proposed Regulations, defines terms used in the Regulations and provide for the application of the proposed Regulations subject to industry-standard tolerances.

##### Pt 2 – Building Code of Australia

Pt 2 provides for the adoption of the BCA and the classification of buildings.

##### Pt 3 – Owner-builders

Pt 3 sets out the information requirements for certificates of consent, application fees for a certificate of consent and the knowledge requirements for owner-builders. It also provides for VBA to keep a register of certificates of consent.

##### Pt 4 – Combined allotments

Pt 4 provides for applications to treat combined allotments as a single allotment. It sets out design requirements and the responsibilities of a MBS relating to combined allotments.

##### Pt 5 – Building permits

Pt 5 relates to building permits, including:

applications for building permits and the prescribed form for applications

exemptions for building permits for a stage of building work

report or consent of reporting authorities

prescribed reporting authorities

the issue of building permits

requirements to make permits and related documents available and to display permit information at the allotment concerned

the duties of a building surveyor in relation to documents, fees and permit details

the duties of a council to keep and make records and information available, and the fees for those requests

time limits for the commencement and completion of building work and building permits

lodgement fees

guarantees and bonds in the case of demolition, removal or re-erection of a building.

##### Pt 6 – Siting

Pt 6 defines terms and imposes siting requirements for particular classes of buildings in relation to:

maximum and minimum street setbacks and side and rear setbacks

building height

site coverage, permeability and private open space

car parking

walls and carports on boundaries

daylight and solar access to habitable room windows

overshadowing and overlooking

fences, masts, poles and Class 10b structures.

Pt 6 allows the relevant council to provide consent to applications for building permits in relation to designs that do not comply with Regulations in relation to the above matters.

##### Pt 7 – Projections beyond street alignment

Pt 7 sets out detailed design requirements relating to projections beyond the street alignment and:

architectural features

windows and balconies

verandahs, sunblinds and awnings

service pipes and rainwater heads

window shutters

signs

service cabinet doors.

The part requires a report and consent of the relevant council must be obtained if any of the above parts of a building project do not comply with the Regulations.

##### Pt 8 – Protection of adjoining property and the public

Pt 8 provides for the safety of adjoining property and the public. It provides for requirements for:

protection work that must be undertaken in respect of an adjoining property and exceptions to carrying out this work

precautions that must be undertaken before and during building work to protect public safety

excavations

retaining walls

demolitions.

##### Pt 9 – Building work

Pt 9 sets out a range of general requirements in relation to:

the testing of materials

fire-performance requirements and fire-safety matters

certificates of compliance

records of pile-driving

branding of timber

stormwater drainage

reports or consents for building over easements, buildings above or below certain public facilities, in relation to septic tank systems, and for electricity substations.

##### Pt 10 – Building work – safety requirements

Pt 10 sets out building work safety requirements for:

barriers for swimming pools and spas including the maintenance and operation of barriers and the new requirement to upgrade certain swimming pool barriers by 1 October 2020

fire-safety requirements including a requirement to install a smoke alarm and sprinkler system in certain existing residential buildings.

##### Pt 11 – Designation of special areas for building work

Pt 11 provides for the designation and mapping of special areas and sets out the requirements in relation to building work in special areas. It also provides for land that is liable to be flooded, or in which buildings are likely to be subject to attack by termites, or are likely to be subject to significant snowfalls.

##### Pt 12 – Bushfire safety

Pt 12 sets out requirements in relation to bushfire-prone areas. It provides for the designation of bushfire-prone areas, for construction standards in designated bushfire-prone areas, requirements for replacing buildings destroyed in the 2009 bushfires and for the construction, maintenance and operation of private bushfire shelters.

##### Pt 13 – Inspection notices and orders

Pt 13 sets out the requirements for mandatory notification stages for the construction or alteration of a building, the demolition or removal of a building and for the construction of swimming pools. It also sets out the information requirements for emergency orders, building notices, building orders and amendments or cancellations of a building order.

##### Pt 14 – Occupancy permits and certificates of final inspection

Pt 14 sets out the requirements in relation to occupancy permits, certificates of final inspection and record-keeping, including:

the prescribed form and information for permits including the requirement to include conditions relating to the maintenance of ESMs on the occupancy permit

making it a condition of an occupancy permit to comply with a maintenance determination if one is issued in accordance with pt 16 of the proposed Regulations

time limits

forms for permits and certificates of final inspection

requirements to display permits and to make permits available for inspection

requirement for a council to keep a register of occupancy records

responsibilities of building surveyors in relation to supplying information to a council and notifying a chief officer of the issue of certificates of final inspection.

##### Pt 15 – Places of public entertainment

Pt 15 prescribes PoPEs for the purposes of the Act and sets out the conditions of use for these places. It also defines prescribed temporary structures and sets out a requirement relating to the design of such structures.

##### Pt 16 – Maintenance of buildings and places of public entertainment

Pt 16 sets out how maintenance requirements for ESMs must be recorded on a maintenance determination and in a maintenance schedule and requires maintenance schedules to be prepared for existing buildings in certain circumstances. It also requires annual reporting of maintenance of ESMs and sets out some general requirements for ESMs including not to remove an ESM from a approved location and to make records available in certain circumstances. It also sets out the obligations of the building occupier with regards to exits.

##### Pt 17– Existing buildings

Pt 17 sets out how current building standards apply to existing buildings and gives a building surveyor power to allow for partial compliance or to grant an exemption from complying with the proposed Regulations when taking into account health and safety and fire spread matters. It also provides for requirements for small, live-music venues.

##### Pt 18 – Building product accreditation

Pt 18 provides for the accreditation of building products, construction methods or designs, components or systems connected with building work including:

prescribing a person or body to accredit building products

applications for accreditation

accreditation and revocation of accreditation

issuing certificates of accreditation, keeping a register of accredited products, record-keeping and confidentiality requirements.

##### Pt 19 – Building practitioners

Pt 19 sets out requirements for:

the period for which a PBS may not in certain circumstances accept an appointment to carry out work in relation to building work

application and renewal of registration including prescribed information that must accompany an application including information as to the character of the applicant

eligibility criteria, conditions on registration, professional standards, disciplinary proceedings and fees

exemptions from registration requirements for certain building practitioners.

##### Pt 20 – Building Appeals Board

Pt 20 sets out:

the prescribed appeals period for appeals to BAB

fees for lodging an appeal, referral application or request to BAB

additional fees for a request for a fast-track appeal and circumstances where those fees are to be refunded.

##### Pt 21 – Infringement notices

Pt 21 provides for the application of infringement notices to contraventions of the Regulations and:

prescribes the authorised officers for the purposes of offences against the Regulations

identifies the prescribed offences for the purposes of the definition of 'prescribed offence' in s 254 of the Act, which relates to building infringement notices

prescribes the penalty for building infringements and how this penalty must be paid.

##### Pt 22 – Exemptions

Pt 22 of the Act provides for exemptions:

from permits and permit application fees, the Regulations, permits for temporary structures and for farm buildings

for building surveyors from notice requirements in respect of certain public buildings.

##### Pt 23 – Transitional provisions

Pt 23 provides for transitional arrangements for the registration of building practitioners and the application of the Act in relation to major domestic building contracts, and it provides for a three-month grace period after the proposed Regulations come into effect.

The transitional provisions will provide that for three months from the start of the Regulations, it is sufficient to comply with certain requirements of the current Regulations. This transitional arrangement will be included to allow users of the building system sufficient time to update their business systems to fully comply with the proposed Regulations.

## 3.3	Summary of cost–benefit analyses

The analysis in the RIS demonstrates that while the scale and scope of the Regulations are significant, they deliver significant benefits to a broad range of stakeholders. The quantified breakeven points are considered to be achievable by the department and it therefore considers the proposed Regulations will deliver net benefits. This is despite a lack of data in some cases either to support or contradict this consideration. Finally, the RIS proposes a range of amendments that will reduce regulatory costs relative to the current Regulations.

Two cost–benefit analyses were carried out at different times and by different consultants:

PwC conducted an analysis of regulatory options other than those related to building practitioner regulation and RIS Part B explains this analysis

Regulatory Impact Solutions conducted an analysis of the options related to building practitioner registration and RIS Part C explains this analysis.

As a result there are some differences in the underlying assumptions of the two cost-benefit analyses, particularly with regard to the hourly rate for practitioners.

RIS Part C costs the rate of a person applying for registration (that is, ordinary earnings of $37.36 per hour for an ordinary person) whereas RIS Part B costs the rate of a person performing obligations under the Regulations (that is, $215 per hour for a PBS and $50 per hour for a MBS or administration staff). In RIS Part B, the higher rates are due to the person being registered as a building surveyor and thus being able to command a higher hourly rate with their skills, and includes business overheads (being overheads that would not exist for an individual attempting to enter the building market).

Where RIS Part C costs the hourly rate for existing practitioners (as distinct from applicants), this rate is also different from the hourly rate in RIS Part B. In RIS Part C, the rate for existing practitioners – $56.04 per hour – refers to an hourly rate for building practitioners in general including people who may only be registered for limited skilled work that results in limited remuneration. By contrast, the rates in RIS Part B are largely attributable to building surveyors and their administration staff.

The department conducted the analysis for the Victorian variation assessed under RIS Part B Appendix C.

In Table 5 and Table 6, the costs are relative to the base case where no Regulations would be made. Also, each option has been costed independently of other options and the combined cost-benefit effect of some options has not been calculated, because the costs and benefits are not additive. For most topics that have been quantitatively assessed in the RIS, a breakeven analysis has been undertaken, due to the difficulty for most Regulations of establishing what would occur in their absence.

In addition, there are some topics for which the costs and benefits were not quantifiable, and they are described qualitatively in the RIS.

Table 5: Cost–benefit analysis for preferred options in RIS Part B: Design, construction, completion / use and other topics

| Topic | - Costs (NPV over 10 years) | Benefits (NPV over 10 years) |
| --- | --- | --- |
| Siting | N/A | $170.6 m |
| Swimming pool barriers | $221.4 m–$395.8 m | $115.6 m–$185.0 m |

Table 6: Breakeven analysis for preferred options in RIS Part B: Design, construction, completion / use and other topics

| Topic | Costs (NPV over 10 years) | Benefit required to justify the costs |
| --- | --- | --- |
| Building permit requirements | $175.4 m–$182.4 m | A time saving of between 8.3%–8.6% to applicants and RBSs due to prescribed information and 670 avoided building defects each year. |
| Reporting authority approvals | $134.3 m | The department considers the benefits of increased flexibility – allowing for appropriate building designs that would otherwise be prohibited from proceeding and reduced risk through preventing high-risk or inappropriate designs in high-risk areas – are likely to outweigh the associated regulatory costs. |
| Protection work | $100.6 m | Reduced delay costs of $369 for each domestic building permit and $738 for each nondomestic building permit. |
| Requirements for inspections, directions, notices and orders | $743.2 m–$1,202.1 m | Reduced building defects targeted by existing mandatory inspections of between 7%–28% with a further reduction in high-risk, high-cost defects targeted by additional mandatory notifications of 6%. |
| Occupancy permits and certificates of final inspection | $96.4 m | A time saving accruing to applicants and building surveyors due to prescribed information of 16%. |
| PoPEs | $10.9 m | One avoided building fire in a PoPE every 0.75 years or one death avoided every 3.25 years. |
| ESMs | $301.8 m–$441.9 m | Reduction in the cost of an average building fire of $700,000–$1.0 million and 4–6 avoided fire-related deaths annually. |

Table 7: Cost–benefit analysis of preferred options in RIS Part C: Building practitioner registration

| Topic | Costs (a year) | Benefits |
| --- | --- | --- |
| Professional standards | N/A | No material cost or benefit measured as the option is considered to assist in the management of disciplinary action but of itself does not change behaviour. The department considers however it provides increased certainty as to why disciplinary actions are being taken. |
| PBSs | N/A | The optimal prescribed period needs to balance the need to avoid conflicts of interest while minimising restricting work where no conflict exists or forcing use of exemption processes. The department considers the current period of one year balances these needs and has no significant impact on the ability of building surveyors to operate. |
| Prescribed information for applicants | N/A | No material cost or benefit as this codifies the practice already used by VBA, for which it has legislative powers. The department considers however that it assists in providing transparency of process for applicants seeking registration as a building practitioner. |

Table 8: Breakeven analysis of preferred options in RIS Part C: Building practitioner registration

| Topic | Costs (a year) | Benefit required to justify the costs |
| --- | --- | --- |
| Practitioner qualifications and experience requirements | $27.0 m | Avoidance of 33,155 detrimental incidents each year, after allowing a cost saving to consumers (reduced search costs) of $2.1 million a year. |
| Prescribed information related to character | $24,171 | Prevention of three unsuitable practitioners being registered each year. |

## 3.4	Competition assessment

Considerations of national competition policy include identifying any restrictions to competition in the preferred option, showing that the restriction is necessary to achieve the objective and assessing whether the benefits of the restriction outweigh the costs in each particular case.

Any new or amending legislation or regulation in Victoria must not restrict competition unless it can be demonstrated that:

the benefits of the restriction, as a whole, outweigh the costs

the objectives of the legislation can only be achieved by restricting competition.

A legislative amendment is considered to have an impact on competition if any of the questions in Table 9 can be answered in the affirmative. The table shows the rationale and significance of those areas where there is an impact on competition.

Table 9: Criteria for determining adverse competition impacts

| Question | - Answer | Significance |
| --- | --- | --- |
| Are the proposed measures likely to affect the market structure of the affected sector(s); that is, will they reduce the number of participants in the market or increase the size of incumbent firms? | Yes | The proposed Regulations may affect the structure of the construction industry and associated industries by limiting the undertaking of certain activities to persons with requisite skills to meet regulatory requirements. For example, some activities can only be undertaken by the RBS, which requires persons to be able to demonstrate they are suitably qualified in order to gain registration with VBA. The department considers it necessary to limit activities (such as assessing and issuing building permits) to suitably qualified persons to ensure building work is performed to the required standard to deliver necessary safety outcomes. |
| Would it be more difficult for new firms or individuals to enter the industry after the imposition of the proposed measure? | Yes | Similar to above, the Regulations bestow certain responsibilities on particular occupations and require individuals to demonstrate they hold the requisite skills and experience to be registered in an occupational category. This limits the ability of individuals to enter the market (as investment is required in developing skills and gaining experience). However, consultations with stakeholders indicate a strong preference to maintain these requirements, given the extreme health and safety risks posed by unqualified persons performing building work or assessing building work for adequacy. The department considers the costs imposed by the Regulations outweigh the associated safety benefits. |
| Would the costs/benefits associated with the proposed measure affect some firms or individuals substantially more than others (for example, small firms or part-time participants in occupations)? | Yes | See discussion of small business impacts. |
| Would the proposed measure restrict the ability of businesses to choose the price, quality, range or location of their products? | Yes | Siting and projection requirements impact on the situation and form of houses. The report and consent process is designed to provide greater flexibility in areas where the Regulations would otherwise prevent a particular design because it is high-risk or in a high-risk area. Where an occupancy permit is required, the Regulations allow the permit to include conditions concerning the use of the facility. |
| Would the proposed measure lead to higher ongoing costs for new entrants that existing firms do not have to meet? | No | The department expects the proposed Regulations will have broadly similar impacts on existing and new entrants into relevant industries. |
| Is the ability or incentive to innovate or develop new products or services likely to be affected by the proposed measure? | Yes | The Regulations impose a range of requirements on buildings and building structures, professionals (such as building surveyors), places (such as for public entertainment), and building products, constructions methods or designs. The preceding chapters demonstrate the imposition of these requirements is the best way of addressing the identified problem/s. |

Note: Questions are from the Government of Victoria 2014, Victorian Guide to Regulation, toolkit 2: Cost–benefit analysis — checklist and alternatives. Available at: http://www.dtf.vic.gov.au/publications/victoria-economy-publications/victorian-guide-to-regulation.

## 3.5	Small business impacts

While not mandatory for a RIS, this section provides high-level analysis of the small business impacts of the proposed Regulations. It outlines the types of small businesses affected and the nature of the effects, including in instances where small businesses are likely to be affected disproportionately.

#### Private building surveyors

PBSs are involved in some aspects of the Regulations including:

assessing and issuing building permits

assessing and issuing occupancy permits including any requirements for ESM maintenance

performing mandatory inspections and issuing directions, notices and orders

assessing the need for protection of adjoining property.

Consultations with PBSs indicate most costs incurred in relation to the building Regulations are generally passed on to applicants and building owners. The cost of forwarding information to VBA on a monthly basis is ultimately born by PBSs, with the magnitude of this cost contingent on whether an automated or manual approach to recording and forwarding information is utilised by the surveyor. The estimated cost associated with this task is between $50 and $200 a month, depending on the approach adopted.

#### Architects

Architects are required to ensure that designs comply with the building Regulations, particularly in relation to siting and projections. This may require designs to be reworked or modified in order to ensure compliance (such as where a report and consent application is rejected). Similar to PBSs, any costs incurred in relation to these activities are generally expected to ultimately be passed on to building permit applicants, including any fees associated with report and consent applications.

#### Builders

Building practitioners incur costs associated with the Regulations relating to:

collating and forwarding documents to the RBS as part of building permit and occupancy permit applications and for certificates of final inspection

notifying building surveyors of mandatory inspection stages, which may potentially incur delay costs if the project is required to stop for a significant period while an inspection occurs

rectifying work in relation to any direction, notice or order.

The estimated cost of collating documents as part of a building permit and occupancy permit application is $200 an application, and for a certificate of final inspection the cost is estimated at $100.

Consultations with building practitioners and building surveyors indicated that building projects are generally planned to avoid delays by providing surveyors adequate notice of a required inspection. Where rectification costs are incurred after a direction, notice or order, this is because building work has been assessed to not be of a required standard to satisfy the surveyor that the work is safe. Therefore, any costs resulting from this process are outweighed by the resulting benefits of ensuring the building work is safe.

#### Small businesses owning buildings required to maintain ESMs

Owners of Class 1B to Class 9 buildings are required to maintain ESMs to ensure they continue to function effectively. Some of these buildings will be occupied by small businesses, particularly:

Class 5 buildings (office buildings for professional or commercial purposes)

Class 6 buildings (shops or other buildings used for the sale of goods or supply of services)

Class 7b buildings (for storage or display of goods or produce for wholesale)

Class 8 buildings (where a process is carried out for trade, sale or gain).

The department considers these costs are necessary, given the significant risk to life posed by an ESM that does not function as designed. For example, if fire-safety measures were to fail:

safe evacuation of a building could be hampered, which could increase the likelihood of injury to or the death of occupants or responding firefighters

the fire might be more difficult to contain, suppress and extinguish, which could increase the extent of losses and other broader impacts on the community and environment.

#### Small business owners requiring report and consent

Small businesses applying to build commercial premises may incur costs in relation to report and consent, particularly for retail buildings with an awning over the street alignment. While minimum heights may impose costs due to a preferred building design not being selected, this is considered necessary given the significant safety risks posed to pedestrians from vehicles striking building projections.

#### Impacts relating to changes to registration requirements

The proposed Regulations make minor changes to the current Regulations for the registration of building practitioners, aiming to standardise the information provided by applicants in order to minimise the need for VBA to request additional information. Nevertheless, further changes to arrangements for registering building practitioners will likely come into effect on 1 July 2017. Any changes will be made following a consultation process that will be undertaken separately from the RIS. This process will not involve another RIS but will include engagement with industry in the form of stakeholder meetings so that industry has an opportunity to comment on the proposed approach to registration (see RIS Part C).

## 3.6	Evidence improvement project

As explained in RIS Part B, some other possible changes to the Regulations are proposed for further consideration by the department. The forward work program as part of the ongoing review of building policy will address whether further regulatory amendments should be made. Submissions on these matters including evidence for the need for regulation or otherwise will help the department determine priorities for and outcomes of the policy review program.

The changes proposed for further consideration are, in summary:

design

  - the scope of matters requiring report and consent

  - expand allowable encroachments related to balconies, carports, garages and buildings underground

construction

  - extend time limits for completion of building work

  - the ongoing need for bonds and guarantees

  - limit the mandatory inspection regime to domestic building work only

  - remove the requirement to have the design of an existing stormwater drainage system approved by a building surveyor

Completion/use

  - remove the requirement to obtain an occupancy permit for prescribed places

  - remove booths with a floor area greater than 100 m2 and include inflatable structures with a floor area greater than 100 m2 in the definition of tents, marquees and booths

  - decrease the threshold floor area to 100 m2 in the definition of stages and platforms

  - include a greater range of structures in the definition of prefabricated buildings

  - increase all penalties and infringeable offences in the Regulations to the maximum allowed

  - include ESM maintenance as a condition on an occupancy permit

  - require the annual ESM report to be certified by a suitably qualified person

  - clarify the definition of owner of a swimming pool or spa

  - establish a voluntary swimming pool register

Builder practitioner registration

  - describe professional standards in more detail

Fees

  - The department is proposing to update the current Regulations related to fees charged by councils and reporting authorities so they reflect full cost recovery and remake the fees for VBA and BAB without change; the department will review all fees within a three-year timeframe (see below).

## 3.7	Regulatory fees review

The current Regulations set council and reporting authority fees relating to:

building permits

reports and consents

lodgement of documents

information requests.

The current Regulations set VBA and BAB fees for applications for:

product accreditation

the issue of duplicate building practitioner certificate

a range of appeal and review applications

site inspections (if the department is advised that a fee for this activity needs to be retained because site inspections may be requested which are not associated with fast-track appeals).

Among the issues about which there is either insufficient data or evidence on which to base a strong argument for regulatory change, is the application of fee structures associated with a council’s discharge of its functions under the Act. Analysis undertaken of a sample of councils indicates a wide range of costs per application. The lack of evidence will be addressed as part of the evidence improvement project explained in Chapter A4.1 Evidence improvement project.

To allow for both aspects of statutory fees to be examined in more detail, the department has proposed that Regulations relating to fees be remade with a three-year expiry date. The review process will commence in 2018–19 with new Regulations to be in place by 5 June 2020. Acknowledging there are very different issues being experienced by councils and VBA, two projects will be undertaken in parallel:

a review of local government fees charged under the Act and Regulations

VBA’s fee structure.

The review of local government fees charged under the Act and Regulations will aim to establish a fee structure that provides for councils to deliver services under the Act and Regulations on a cost-recovery basis. Councils and stakeholders will be involved early in the process. The review will gain insight into the processes, systems, resources and cost structures councils apply to deliver building-related services. The review will take into account the differing characteristics of municipalities. The process will draw on the outcomes of the data-collection process, and so early engagement with councils will precede the formal review.

The review of VBA’s fee structure will focus on meeting the costs of implementing the recent building reforms. It will consider the efficiencies derived from establishing a modern information technology platform to manage business information requirements and consideration of fees and charges relating to the registration and building practitioner conduct functions that have been transferred from the now-abolished Building Practitioners Board. The department will work with VBA to ensure timely consideration of any necessary changes to Regulations.

# 4	Evaluation

In accordance with the requirements of the Victorian Guide to Regulation, a midterm evaluation of the proposed Regulations will aim to:

determine the extent to which the objectives of the proposed Regulations have been achieved

identify changes that may be needed to better achieve those objectives or other objectives that emerge in the intervening period.

During the first three years of the proposed Regulations (2017–19), the department will monitor their implementation. This will start with an evidence identification and collection process that will establish a baseline dataset to build a more robust evidence base and inform any future regulatory changes. Regular and improved evidence collection will assist in early identification of emerging issues that require a regulatory response before the midterm evaluation.

The primary function of the midterm evaluation is to see if the proposed Regulations are delivering better outcomes than those now being delivered by the current Regulations.

The evaluation will take account of factors that may affect building system outcomes other than the Regulations (such as the effect of broader legislative reforms and changes in the economic environment), acknowledging that the proposed Regulations are only one component of the regulatory framework in which the industry operates. To better understand these factors (that are important to the overall evaluation), the approach in the midterm evaluation will aim to:

enable stakeholders to comment on the effective working of the Regulations and raise any issues relating to the practical implementation of new Regulations or to identify aspects that require further investigation; the evidence-collection process will be a critical component in providing a more-rigorous evidence base to support any future regulatory change

assess the impact of legislative reforms in the Building Legislation (Building Enforcement and Other Measures) Bill 2016, to determine whether further amendment is required to the Regulations to respond to new legislative provisions.

Table 10 sets out the proposed issues of interest for the midterm evaluation.

Table 10: Overview of issues and objectives for the midterm evaluation

| Issues being addressed by proposed Regulations | Objectives of proposed Regulations to be tested in the midterm evaluation |
| --- | --- |
| The significant information asymmetry that exists between the building industry (including consumers); building owners; people who live, work in and use buildings; and regulators. | Reducing complaints and disputes about building practitioners, defective and/or noncompliant building work and buildings |
| The long-term risk to the health and safety of building occupants that arises when building owners or building practitioners are motivated to reduce costs in the short term | Reducing the amount of building work found to be noncompliant or defective by RBSs, MBSs and regulators |
| The low levels of compliance with maintenance requirements for existing buildings | Improving accountability for the maintenance of ESMs Improving building owners' compliance with requirements to maintain ESMs and swimming pool and spa barriers |
| Reducing regulatory burden | Increasing the efficiency of the administration of building permits Reducing regulatory red tape for PoPEs without increasing risks to public health and safety Reducing regulatory red tape in the commercial building sector where information asymmetry is not a major concern for industry participants Reducing the number of planning approvals required for single dwellings through more effective siting Regulations |

The evaluation strategy is sensitive to the dynamic environment within which the building Regulations are being reviewed. As indicated in earlier chapters, the review of the building Regulations is being undertaken at a time of substantial change in the building industry.

Consequently, the evaluation strategy is intended to be a live document that will be reviewed by the department with the assistance of VBA on a regular basis to ensure it remains relevant to the incremental changes occurring through the broader industry reform program. The review of the evaluation strategy will also be informed by stakeholder input, taking into account feedback obtained through the RIS consultation period and through the release of discussion papers on priority topics. As part of the consultation process for the RIS, the department will hold stakeholder roundtable sessions to seek preliminary comment on directions within the RIS. This will assist in formulating a preliminary view on aspects that will require further consideration in the remaking of Regulations.

It is intended also that a minimum of two policy papers on priority topics will be developed each year in the three-year period, starting in 2017–18. The department will work with VBA and BRAC to determine the scheduling of policy topics and the timing of the release of policy discussion papers.

This will enable a systematic and efficient approach to assess the effectiveness of Regulations. To coincide with the remaking of the Regulations, the department will in July 2017 commence development of a regulatory framework plan that will help:

confirm priority actions to be commenced within the first 12 months of the implementation of the proposed Regulations

define the outcomes to be achieved within the three-year cycle

inform and refine the evaluation methodology for the five-year review of the proposed Regulations

Stakeholders will be consulted about the regulatory framework before it is finalised.

## 4.1	Evidence improvement project

The evidence improvement project is a cornerstone of the evaluation strategy. It has been triggered by the lack of robust data with which to evaluate some aspects of the current Regulations and to assess the validity of proposals for change put forward by stakeholders during the sunsetting review.

The effectiveness of the midterm evaluation will require:

working with stakeholders to establish an evidence base that will improve the quality of information to underpin assessment of the effectiveness of Regulations including assessment of any need for regulatory change

obtaining access to data and information from a range stakeholders in a consistent and regular form to test how well the Regulations have achieved the issues identified in Table 10 above.

### 4.1.1	Key performance indicators and baseline data

The department will work with VBA, councils and fire authorities to develop key performance indicators (KPIs) as part of the stakeholder engagement process, to obtain more robust evidence. These KPIs will be discussed with the Office of the Commissioner for Better Regulation before finalisation of the minimum dataset and the collection of baseline data to ensure as far as practicable acceptance of the proposed future dataset. This will also allow for discussion about any required interim dataset, given there may be a requirement on the part of stakeholders to modify collection systems or obtain prior agreement to acceptable proxies to supplement any identified data gaps that cannot be readily met.

Data will be collected about two years before the midterm evaluation, based on agreed database structures and data-collection protocols developed as part of the evidence improvement project.

### 4.1.2	Evidence collection to support the midterm evaluation

The data-collection process will commence in 2017–18 with the identification of a core set of baseline data that will be required in order to assess the effectiveness of the Regulations.

At a minimum, data for the evaluation will be drawn from sources including:

VBA’s systems for empirical data related to complaints, building permits and compliance and enforcement activities

BAB data systems

councils for data related to building permits and building inspection outcomes

fire authorities for data related to fire-safety requirements and ESMs

building surveyors for data about the effectiveness of streamlined building permit information requirements

CAV.

The midterm evaluation will also draw on:

qualitative data from stakeholder consultations about the strengths and weaknesses of the proposed Regulations

the views and experience of BRAC

the findings of VBA’s research and evaluation activities

the views of councils about the effectiveness of the revised siting protocols

consumer and industry feedback provided to CAV, councils and VBA about the proposed Regulations.

As indicated above, the department will ensure the midterm evaluation is completed and liaise with OCBR about the adequacy and transparency of the evaluation. The evaluation is expected to occur over a period of at least six months in order to allow sufficient time for stakeholder consultation, data collection and analysis.

The evaluation strategy will be undertaken in conjunction with VBA with input from VBA’s Research and Evaluation Unit. The unit aims to work towards developing a strong evidence base and improving the functioning of the building and plumbing regulatory system. Research and evaluation projects will aim to identify and explore key regulatory problems, issues and potential solutions.

Findings and outcomes of the program will enable VBA to:

better understand key regulatory problems including risks, harms, causes and unintended consequences

target areas of greatest concern and opportunities for regulatory improvement to minimise harms

evaluate the effectiveness and efficiency of existing regulatory interventions

inform future policy directions and recommend feasible solutions

adopt earlier, more-targeted interventions with an increased likelihood of success and return on investment.

### 4.1.3	Evaluation strategy program

#### Year one (2017–18)

The department will (in the 12 months after the start of the proposed Regulations) work with VBA, councils and industry stakeholders to identify and agree on the core dataset required to enable a more robust assessment of the need for future regulatory change. This is the evidence improvement project.

Engagement from key stakeholders will be fundamental to ensuring that future data collection will be efficient, relevant and consistent and that issues of data collection, management and retrieval will be addressed concurrently. The evidence improvement project is explained in more detail in Chapter A4.1 Evidence improvement project.

In 2017, the department will develop new Regulations to implement the government’s second stage of legislative reforms. This work will commence concurrently with the government’s consideration of proposed legislative reforms. Targeted consultation will be undertaken on any proposed Regulations. The Building Regulations Advisory Committee will be a principal mechanism to test any proposed Regulations.

#### Years two and three (2018–19)

The regulatory review process has highlighted there is a need to further examine the fee structure to determine appropriate costs associated with a council's discharge of its functions under the Act. VBA has commenced a review of its fee structure that will take into account the impact of recent and ongoing reforms to the building system on the cost of carrying out statutory functions. VBA’s review will not be concluded within the timeframe of this regulatory review.

The department proposes to make Regulations relating to fees with a three-year expiry date. A review process will commence in the 2018–19 year with new Regulations to be in place by 5 June 2020 covering the review of:

council and reporting authority fees prescribed under the Regulations relating to building permits, reports and consents, lodgement of documents and information requests.

VBA and BAB fees relating to applications for product accreditation, duplicate building practitioner certificates, a range of appeal and review applications and site inspections (if BAB advise that this needs to be retained as a separate fee, because in the proposed Regulations this fee has been removed as the department understands site visits are only undertaken in relation to fast-track appeals).

VBA is currently undergoing its own change management review to put into place new processes and practices that will respond to the building reforms including consideration of fees and charges relating to the registration and building practitioner conduct functions that have been transferred to it from the now-abolished Building Practitioners Board. The department will work with VBA as its internal review progresses to ensure timely consideration of any necessary changes to Regulations.

The evaluation strategy will be adapted as necessary to respond to emerging directions following parliament’s consideration of further legislative reforms, as well as to emerging issues.

Acknowledging that work is already underway to improve data identification, the department with the assistance of VBA, will:

convene a small, stakeholder-nominated working group to develop a minimum dataset to inform future data collection with the aim of finalising requirements by 30 June 2018

establish a stakeholder collaboration strategy to tackle data identification and collection issues, that will set future expectations of data management accountabilities; at a minimum, at least two stakeholder forums will be held to enable consideration of issues from across the breadth of industry participants

continue engagement with the Office of the Commissioner for Better Regulation to ensure expectations in relation to data capture are identified and resolved before finalising the minimum data collections

establish a consistent reporting mechanism that enables early identification of data gaps, collection limitations or any other limiting factors to enable issues to be remediated early in the collection process.

This approach will enable consideration of practical strategies to enable the development of better evidence-collection practices as industry stakeholders change their processes and practices in response to broader building reforms.

# 5	Regulation change summary

Table 11: What the column headings mean

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| The Regulations in this column correspond to the (current) Building Regulation 2006 | This column will be chronological, based on the numbering of the proposed Regulations | The text in this column describes what the proposed Regulations would do. | The ticks in these columns are explained in the next table below |  |  |  |  | The text in this column explains why the Regulation is proposed. If a current Regulation has changed, this change is described and where applicable the reason for the change has been explained. |

Table 12: What the ticks in the 'Proposed regulatory change' column mean

| Regulatory change – this table explains what a tick in the columns below means |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revoke | No change | Updated | Reduce | Strengthen |
| The current Regulation will not be remade as part of the proposed Regulations. | The current Regulation will be remade with no substantial change to its operation, but cross-references will be updated and the language and structure of the Regulation will be modernised. | The current Regulation will be remade with one or more remedial amendments to address an issue with its operation (such as ambiguity). | The current Regulation is amended to reduce the burden (compliance, administrative and other costs) associated with the operation of the equivalent Regulation. | A current Regulation has been amended or a new Regulation introduced, to respond to new or emerging issues. |

### Pt 1—Preliminary

#### Div 1—Introduction

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 101 | 1 | Objectives States the objectives of the proposed Regulations. |  | ✓ |  |  |  | An objectives clause must be included in all Regulations. The Regulations deal with matters within the scope of regulatory objectives. |
| 102 | 2 | Authorising provisions States the provisions of the Building Act 1993 (the Act) which authorise the making of the Regulations. |  | ✓ |  |  |  | This Regulation cites the general regulation-making powers in the Act that support the making of the proposed (2017) Regulations. |
| 103 | 3 | Commencement The commencement clause specifies the day on which the Regulations would come into operation. This Regulation specifies the Regulations are to come into operation on 1 June 2017. |  |  | ✓ |  |  | This Regulation is required because the Regulations will come into operation on a day after they are made by the Governor in Council. Proposed reg 280 provides a grace period of three months to either comply with the current Regulations or the proposed Regulations. |
| 104 | 4 | Revocation Revokes the current Regulations and each subsequent set of amending Regulations. |  |  | ✓ |  |  | This Regulation is needed because the Regulations would replace an earlier set of principal Regulations, being the Building Regulations 2006. |

#### Div 2—Definitions and interpretation

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 105 | 5 | Definitions Sets out any definitions used more than once in the proposed Regulations. |  |  | ✓ |  |  | Definitions are used in the Regulations as a shortening device or to remove ambiguity. New definitions are proposed (for example, of 'architectural feature', 'commencement date', 'completion date', 'Keeper of Public Records', 'medium street', 'narrow street', 'north-facing window', 'recreational private open space' and 'wide street'). |
| N/A | NEW 6 | Interpretation Provides that the proposed Regulations prevail over any incorporated document where any inconsistency arises between the two. |  |  |  |  | ✓ | This Regulation ensures any document incorporated in the Regulations that is amended after incorporation, and in a manner inconsistent with the intended operation of the proposed Regulations, does not impact on the operation of the Regulations. |
| 107 | 7 | Numerical values Specifies the application of numerical values prescribed in the proposed Regulations, allowing for certain accepted tolerances when applying these values to building work. |  | ✓ |  |  |  | This Regulation allows reasonable flexibility in the application of building standards according to industry-accepted practice. |
| 108 | 8 | Use of buildings Clarifies that a reference to the use of a building is also a reference to the purpose for which the building is intended to be used. |  | ✓ |  |  |  | Ensures building standards are applied from the design stage through to the construction and use of a building or PoPE. |
| 1901 | 9 | Approved forms Enables VBA to prepare and approve forms for the purposes of the Regulations. |  | ✓ |  |  |  | VBA is given power to produce forms for the purposes of the proposed Regulations. This provides flexibility as approved forms can be quickly prepared and easily amended in response to changing circumstances. |

### Pt 2—Building Code of Australia

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 109 | 10 | BCA Adopts the BCA so that it forms part of Victorian law. The BCA applies in Victoria, as modified by the Regulations. |  | ✓ |  |  |  | This Regulation supports the implementation of a nationally consistent technical, performance-based building code. |
| 106 | S | Use of BCA terms Specifies when words and expressions in the Regulations have the same meaning as they have in the BCA. |  | ✓ |  |  |  | This Regulation promotes consistency in the meaning of terminology used in both the Regulations and the BCA, in circumstances where the Regulations do not specify a different meaning to that used by the BCA. |
| 112(1) | 12 | Classification of buildings Requires buildings to be classified using the BCA. Classification of a building or part of a building is determined by the purpose for which it is designed, constructed or adapted to be used. |  | ✓ |  |  |  | Building standards and safety and other requirements in Regulations are associated with building classification in the BCA. For this reason, it is necessary for a building surveyor to classify a building when issuing a building permit. |
| 112(2) | 13 | RBS must classify building in case of doubt Directs how difficult building classifications must be resolved and made. |  | ✓ |  |  |  | This Regulation requires the RBS to classify a building as the BCA class it most closely resembles, in situations where more than one BCA building classification may be appropriate for a building or a building does not fit neatly within an existing BCA classification. |

### Pt 3—Owner builders

#### Div 2—Applications for certificate of consent for owner-builders

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 201 | 14 | Applications for certificate of consent Specifies the information that must be included in all applications for an owner-builder certificate of consent. |  | ✓ |  |  |  | This Regulation supports the Act’s operation by providing the detail to give effect to s 25C(2)(a) of the Act. |
| 202 | 15 | Information to accompany application for certificate of consent Prescribes the information which must accompany an application for an owner-builder certificate of consent. |  | ✓ |  |  |  | This Regulation supports the Act’s operation by providing the detail to give effect to s 25C(2)(c)(i) of the Act. |
| 203 | 16 | Additional information to accompany application for certificate of consent if land is owned by body corporate Specifies the additional information that must be included if a company owns the land the application relates to. |  | ✓ |  |  |  | Additional information requirements apply to land owned by a body corporate to give effect to s 25C(2)(c)(i) of the Act to enable VBA to make a decision about the application for a certificate of consent. |
| 204 | 17 | Additional information to accompany application for certificate of consent if land is subject to trust Specifies the additional information that must be included if the land the application relates to is subject to trust. |  | ✓ |  |  |  | Additional information requirements apply to land subject to a trust to give effect to s 25C(2)(c)(i) of the Act to enable VBA to make a decision about the application for a certificate of consent. |
| 205 | 18 | Fee for an application for certificate of consent Prescribes the application fee that must accompany an application, which is 6.90 fee units. |  | ✓ |  |  |  | This supports the Act’s application scheme to obtain an owner-builder certificate of consent by requiring a fee to be paid for the purpose of s 25C(2)(c)(ii) of Act. |
| 206 | 19 | Required knowledge for owner-builders Specifies the knowledge an applicant must be able to demonstrate before a certificate of consent can be issued. This Regulation is supported by sch 10 of the proposed Regulations. |  | ✓ |  |  |  | This is required to give effect to s 25E(1)(ab) of the Act. |

#### Div 2—Certificates of consent

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 203 | 20 | Information in certificate of consent Specifies the details which VBA must include in a certificate of consent. |  | ✓ |  |  |  | This Regulation give effect to s 25G(1)(a) of the Act. It ensures all necessary information is contained in the certificate to support enforcement and monitoring. |
| 204 | 21 | Register of certificates of consent Specifies the information which must be contained in the register of certificates of consent. |  | ✓ |  |  |  | This Regulation gives effect to s 25H(2) of the Act. It enables VBA to monitor and audit building work undertaken by owner-builders. |
| 205 | 22 | Publication of information kept on the register of certificates of consent Specifies the information VBA may publish in relation to the details held in the register of certificates of consent. |  | ✓ |  |  |  | This Regulation gives effect to s 25H(3) of the Act. It reduces search costs by enabling the public to access information to determine if a building has been subject to building work carried out by an owner-builder. |

### Pt 4—Combined allotments

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 501 | 23 | Application Specifies the buildings that are required to comply with combined allotment provisions. |  | ✓ |  |  |  | This Regulation is a mechanical provision. |
| 502 | Multiple | Has been replaced by multiple Regulations. Discussed below in relation to proposed Regulations 28, 29 and 30. |  |  |  |  |  |  |
| n/a | NEW 24 | Application to treat combined allotments as one allotment States who can apply for a combined allotments determination and to whom that application must be made, and how that application must be made. |  |  |  |  | ✓ | This new Regulation provides a clear application process and certainty for all involved. This process is absent from the current Regulations. |
| n/a | NEW 25 | Information to be contained in or accompany application Contains the minimum necessary information which must be included in and accompany an application for the purpose of reg 24. |  |  |  |  | ✓ | This is a new Regulation which is necessary to facilitate efficient decision-making and reduce or avoid delays caused by lack of information. |
| n/a | NEW 26 | Building surveyor may request more information Empowers the building surveyor asked to make a combined allotments determination to request any additional information from an applicant which is required to make a combined allotments determination. |  |  |  |  | ✓ | This is a new Regulation which ensures a building surveyor can request further information to make a determination, if the minimum mandatory information required is insufficient. |
| n/a | NEW 27 | Application may be refused if information not provided Provides that if additional information is requested and not provided, the MBS or the PBS may refuse the application. |  |  |  |  | ✓ | This is a new Regulation to clarify an application can be refused due to lack of information. |
| 502(1) 502(2) | 28 | Determination to treat combined allotments as one allotment Establishes the circumstances when a MBS or a PBS can make a determination in relation to combined allotments. |  |  |  |  | ✓ | Improves efficiency of decision-making by specifying the process and decision-making criteria for making a determination. |
| 502(3) | 29 | Effect of determination For clarity, states the effect of the making of a combined allotments determination which is that the relevant allotments are treated as one allotment for the purposes of the Act and the Regulations. |  | ✓ |  |  |  | Allowing multiple allotments in certain circumstances to be treated as one allotment provides an opportunity to reduce regulatory burden on landowners. For example, by allowing building work to be authorised under one building permit, and also where appropriate to streamline some building standards which would otherwise be required by reason of the existence of an allotment boundary. |
| 502(5) | 30 | PBS may exercise powers of MBS under this part Empowers a PBS to make a combined allotments determination in circumstances where they have been appointed under pt 6 of the Act to issue a building permit for building work in relation to which the determination is sought. |  | ✓ |  |  |  | Removes a limitation on who is empowered to make combined allotment determinations to include a PBS in certain circumstances, in addition to a MBS. |

### Pt 5—Building permits

#### Div 1—General

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1801 | 31 | Exemptions from building permit Prescribes the circumstances in which an exemption from the requirement for a building permit applies. |  | ✓ |  |  |  | This Regulation reduces burden in relation to low-risk / minor building work. |

#### Div 2—Applications for building permits

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 301 | 32 | Applications for building permits Specifies the application form to be used to apply for a building permit and the information to be included in a building permit application. |  |  | ✓ |  |  | This Regulation is necessary to facilitate efficient decision-making and to reduce or avoid delays caused by a lack of information being submitted in an application to support the assessment of compliance of proposed building work with the Act and the Regulations. For administrative efficiency, the use of the prescribed application form has been made mandatory. |
| 302 303 | 33 | Application for permit to construct or alter building Specifies the type and number of copies of documents required to accompany a building permit application, if that application relates to the construction or alteration of a building. |  |  |  | ✓ |  | This Regulation is necessary to facilitate efficient decision-making and to reduce or avoid delays caused by a lack of information being submitted in an application. To reduce regulatory burden, only one hard copy of the documents need be provided with an application if the documents are lodged electronically, which is subject to the discretion of the RBS. |
| 304 | 34 | Application for permit to demolish or remove building Specifies additional information that must be lodged with an application to demolish or remove a building. |  |  |  | ✓ |  | This Regulation is necessary to facilitate efficient decision-making and to reduce or avoid delays caused by a lack of information being submitted in an application. To reduce regulatory burden, only one hard copy of documents need be provided with an application if the documents are lodged electronically, which is subject to the discretion of the RBS. |
| 306 | 35 | Exemption from application requirements for certain building work Empowers the RBS to exempt an applicant from the requirement to lodge certain documents where those documents are not considered by the RBS to be necessary or appropriate to assess compliance with the Act or the Regulations of proposed building work or a stage of proposed building work. |  |  | ✓ |  |  | This Regulation avoids the burden of lodging certain documentation with a building permit application which is not necessary for the RBS to assess compliance of proposed building work with the Act and the Regulations. |
| n/a | NEW 36 | Application for permit where swimming pool or spa proposed Requires an application to demonstrate that a compliant barrier will be constructed in conjunction with the swimming pool. A building permit cannot be issued for the construction of a swimming pool unless a compliant barrier is approved as part of that permit application. |  |  |  |  | ✓ | This Regulation is required to reinforce the safety imperative that a compliant safety barrier is constructed in conjunction with a swimming pool. |
| 305 | 37 | Additional information to accompany application for permit Specifies additional information which a RBS may require to accompany an application for a building permit. |  |  | ✓ |  |  | The power for the RBS to request more information from an applicant to support a building permit application is already available in sch 2 to the Act. Unlike the Act, however, this Regulation provides specific examples of the types of information which a building surveyor may seek to rely on to assess a particular building permit application. This Regulation facilitates efficient decision-making and reduces or avoids delays caused by a lack of information being submitted to support the assessment of compliance of proposed building work. |

#### Div 3—Report or consent of reporting authorities

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 308(1) | 38 | Prescribed reporting authorities Specifies who is a reporting authority in relation to an application for a building permit. |  |  |  |  | ✓ | This Regulation is required to give effect to cl 7(2) of sch 2 to the Act. Service authority has been included to ensure their recommendations are treated as mandatory. |
| 308(2) | 39 | Prescribed matters Specifies the matters that, if a reporting authority identified in reg 38 is required by the 2017 Regulations to report on, any recommendation made in that report must be implemented by the RBS when deciding a building permit application (see also sch 4). |  | ✓ |  |  |  | This Regulation gives effect to cl 7(2) of sch 2 of the Act. |
| 307 | Multiple | Regulation 307 has been replaced by multiple Regulations. These new replacement Regulations are discussed immediately below in relation to proposed Regulations 40, 41, 42 and 43. These proposed Regulations also replace sch 3 of the current Regulations. |  |  |  |  |  |  |
| Sch 3; item 1 | 40 | Time limits for application to be given to reporting authority Specifies the time period within which the RBS must provide a copy of a building permit application to each required reporting authority. |  | ✓ |  |  |  | This Regulation is required to give effect to cl 4(2) of sch 2 to the Act. This requirement was previously contained in a schedule to the current Regulations and has now been included in the body of the Regulations for clarity and ease of reference. |
| Sch 3; item 6 | 41 | Further time for an applicant to provide a report or consent Specifies the prescribed further time period within which an applicant for a building permit must provide a copy of a report or consent to the RBS. |  | ✓ |  |  |  | This Regulation is required to give effect to cl 5(4) of sch 2 to the Act. This requirement was previously contained in a schedule to the 2006 Regulations but has now been included in the body of the Regulations with other related Regulations, for clarity and ease of reference. |
| Sch 3; item 2 Item 3 Item 4 | 42 | Time limits relating to the report or consent of reporting authorities Specifies how a building permit application should progress if a reporting authority fails to provide a report or consent within the specified time in this Regulation. |  | ✓ |  |  |  | This Regulation is required to give effect to cls 6(1), (2) and 3, and cl 6A(2), (3) and (4) of sch 2 to the Act. This requirement was previously contained in a schedule to the 2006 Regulations and has now been included in the body of the Regulations for clarity and ease of reference. |
| Sch 3; item 5 | 43 | Time within which RBS must decide application for building permit Specifies the time period within which the RBS must decide an application for a building permit. If the application is not decided within the time prescribed in this Regulation, the application is deemed to have been refused by the RBS. |  | ✓ |  |  |  | This Regulation is required to give effect to ss 19(2) and 19(3) of the Act. It also clarifies when the relevant time period commences in circumstances where multiple reports or consents are required in relation to a building permit application. This requirement was previously contained in a schedule to the 2006 Regulations and has now been included in the body of the Regulations, with other related Regulations, for clarity and ease of reference. |
| 312 | 44 | Maximum fees for report and consent Specifies fees payable to a reporting authority for consideration of a building permit application when referred to it under the Act or the Regulations. |  |  | ✓ |  |  | This Regulation is required to cover a reporting authorities costs of assessing and responding to a building permit application which has been referred to it. All fee units are updated to correspond with full cost recovery. There are four fee changes (demolitions has changed from 4.60 to 5.75 fee units; siting, allotments projections and work in special areas has changed from 18.43 to 19.61 fee units; protection of the public has changed from 18.43 to 19.90 fee units and stormwater drainage has changed from 4.60 fee units to 9.77 fee units). |

#### Div 4—Building permits

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 313 | 45 | Issue of building permit Specifies the form a building permit must be issued in, the requirement for that form to be signed by the issuing RBS and also information which must be included in a building permit. |  |  | ✓ |  |  | This Regulation is required to improve administrative efficiency. Minor changes have been made to the prescribed building permit form to require more information to be included in the permit in relation to the nature and method of approval of performance solutions authorised by the RBS as part of an approved scope of building work. |
| n/a | NEW 46 | Building surveyor to document assessment of performance solutions Requires the RBS to record in writing how performance solutions have been assessed when issuing a building permit. |  |  |  |  | ✓ | This Regulation addresses information asymmetry and is a new Regulation to improve record-keeping of potentially higher risk assessment decisions. |
| 314 | 47 | Building surveyor to provide copy of permit Requires the RBS to forward a building permit and approved documentation to the applicant, within seven days of its issue. |  |  |  |  | ✓ | This Regulation is required to ensure that an applicant is provided with a copy of the authorisation (building permit), as soon as practicable after it is issued, to allow building work to commence. It has been updated to allow documentation to be forwarded electronically by the RBS, as well as to address information asymmetry by ensuring that copies of permits are forwarded to relevant parties. |
| 315 | Multiple | Has been restructured into multiple Regulations. Discussed in relation to proposed Regulations 61, 62, 63 and 67. |  |  |  |  |  |  |
| 316 | 48 | Applicant to make documents, permit available Requires the applicant for a building permit to make available to the person responsible for carrying out the approved building work any approved plans (and other documentation) necessary to carry out that building work. |  | ✓ |  |  |  | This Regulation is required to addresses information asymmetry. Its purpose is to ensures that the builder is provided with the approved and endorsed construction plans before construction is commenced. |
| 317 | 49 | Provision and display of permit information Requires that the person in charge of the building work keeps onsite, and available for inspection, a copy of the building permit and approved plans, specifications and documentation for the building work, in addition to registration and permit number details. |  |  |  |  | ✓ | This Regulation is required to address information asymmetry and to facilitate regulatory monitoring and enforcement. The obligation to have a copy of the building permit and related documentation onsite is now drafted as a strict requirement which will assist the enforcement of this offence. |
| 318 | 50 | Owner to notify change of owner or builder Requires the owner to notify the RBS of any change in name or address of the owner or builder, within 14 days of such a change. |  | ✓ |  |  |  | This Regulation is required to addresses information asymmetry. Its purpose is to ensures that the RBS has the current contact details of the owner and builder for communication and enforcement purposes. |
| n/a | NEW 51 | Builder to notify change of name or address Requires a building practitioner nominated in a building permit to notify the RBS of any change to their name and/or address. |  |  |  |  | ✓ | This Regulation is required to address information asymmetry. It ensures that the owner and the RBS have the current contact details of registered building practitioners responsible for building work authorised by a building permit for monitoring and enforcement purposes. |

#### Div 5—Duties of the building surveyor

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 319 | 52 | Documents to be given to council Prescribes the documentation, in relation to a building permit, that the RBS must lodge with council for record-keeping purposes. |  |  |  |  | ✓ | This Regulation is required to give effect to s 30(1A) of the Act. New forms have been added to allow building surveyors to better document and record their assessment and compliance decisions. These forms have been added to the list of documents required to be lodged by the RBS with the relevant council. |
| 320 | 53 | Lodgement fees Prescribes the fee for lodging building permit documents with council. |  |  |  |  | ✓ | This Regulation is required to give effect to cl 1(d) of sch 2 of the Act and to provide for an exemption from paying the fee in relation to classes of building referred to in proposed reg 277. The exemption for work valued at below $5,000 is removed to reflect the preferred option of full cost recovery. It is currently 2.75 fee units and is proposed to be changed to 8.23 fee units. |
| 321 | 54 | Building permit levies Requires the RBS to provide monthly reports to the VBA on matters relating to the collection of the building permit levy. |  | ✓ |  |  |  | This Regulation is required to give effect to s 205J of the Act. Its purpose is to address information asymmetries and facilitate the efficient and effective collection of the building permit levy. |
| 322 | 55 | Information the RBS must give to VBA Requires the RBS to provide targeted information to the VBA, on a monthly basis, in relation to the progress of key stages of building work authorised by the RBS by means of a building permit. |  |  |  |  | ✓ | This Regulation is required to address information asymmetries and to assist the VBA in its monitoring and enforcement functions under the Act. It now requires additional reporting requirements of RBSs than previously. |
| 323 | 56 | Guarantees and bonds Specifies the circumstances in respect of which the RBS may require a guarantee or bond to secure the completion of building work and also the amount of the security. |  |  |  |  | ✓ | This Regulation is required to give effect to s 22 of the Act. The required security amount for re-erection has been increased from the amount set over 10 years ago, to ensure the costs of rectification can be covered should this be required. |

#### Div 6—Duties of council

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 324 | 57 | Period that documents must be kept Requires council to hold records of all documents lodged in relation to issued building permits. |  |  | ✓ |  |  | This Regulation is required to addresses information asymmetry. It aligns the requirement to hold records with the obligations council have under the Public Records Act 1973. |
| 325 | 58 | Council to make documents available on request Requires council to make building permit documents available to the owner or mortgagee in respect of any building or land they have an interest in. |  | ✓ |  |  |  | This Regulation is required to addresses information asymmetry for prospective or subsequent owners of a building. |
| 326 | 59 | Requests for information from relevant council Specifies the documents any person can request from the relevant council in relation to any building or land. |  | ✓ |  |  |  | This Regulation is required to addresses information asymmetry for prospective or subsequent owners of a building. The information that can be requested has been broadened. |
| 327 | 60 | Fees for requests for information Prescribes the application fee which must accompany a request for information. |  |  | ✓ |  |  | This Regulation is required to subsidise the cost to councils to respond to requests for building records and information. The fee unit amount is proposed to be updated to allow for full cost recovery. It is currently 3.67 fee units. It is proposed to be changed to 3.19 fee units. |

#### Div 7—Time limits for building work and permits

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 315(1) | 61 | Commencement of work Specifies when building work authorised by a building permit must commence. |  | ✓ |  |  |  | This Regulation is required to mitigate the risk of a design not being in accordance with the most up-to-date requirements due to changing building standards. Regulatory time limits also provide a level of project certainty (outside the terms of a building contract). |
| 315(2) 315(3) | 62 | Completion of building work Specifies when building work authorised by a building permit must be completed by. |  | ✓ |  |  |  | This Regulation is required to mitigate the risk of building work not being in accordance with current building standards. Regulatory time limits also provide a level of project certainty (outside the terms of a building contract). |
| 315(4) | 63 | Lapsing of building permit Specifies when a building permit will lapse in relation to building work authorised by a building permit and the effect on a permit when it has lapsed. |  |  | ✓ |  |  | The purpose of this Regulation is to provide incentive for an owner and a builder to ensure building work is commenced and completed within regulatory time limits. |
| n/a | NEW 64 | Notice of imminent lapse of building permit – commencement of work Requires the RBS to issue a notice to the applicant of a building permit of the date building work must be commenced by, if the RBS has not conducted the first mandatory inspection of building work within three months of the required commencement date. |  |  |  |  | ✓ | This Regulation is required to help ensure a building permit is not left to lapse inadvertently. It improves administrative efficiency because the notice gives the owner enough time to take action to either commence work or to apply for an extension to the commencement date for building work, before the permit lapses. |
| n/a | NEW 65 | Notice of imminent lapse of building permit – completion of work Requires the RBS to issue a notice to the applicant of a building permit of the date building work must be completed by, if the RBS has not conducted the final mandatory inspection within 3 months of the completion date. |  |  |  |  | ✓ | This Regulation is required to help ensure a building permit is not left to lapse inadvertently. It also improves administrative efficiency because the notice gives the owner enough time to take action to either complete work or to apply for an extension to the completion date nominated for the building work, before the permit lapses. |
| n/a | NEW 66 | Copy of notice of imminent lapse of building permit to be provided Requires the RBS to give a copy of any notice issued under proposed new Regulations 64 and 65 to the owner (if not the building permit applicant), the builder, the relevant council and to the VBA. |  |  |  |  | ✓ | This Regulation ensures compliance with building work and the regulatory time frames under the Act and Regulations, by making relevant parties aware of any potential issues. |
| 315(4) | 67 | Extension of building permit Enables an application to be made to the RBS for an extension to either the required commencement or completion dates for approved building work. |  |  | ✓ |  |  | This Regulation is required to allow an owner to avoid costs associated with a lapsed permit by seeking an extension of a relevant regulatory time limit. Current Regulation 315(4) has been revised to clarify the process to apply for such an extension. |

### Pt 6—Siting

#### Div 1—Introduction

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 401 | 68 | Application This Regulation exempts the operation of pt 6 to building work that has had the same matter assessed under the planning scheme. |  |  | ✓ |  |  | This Regulation removes duplication with the planning system. It has been updated to clarify that the exemption only applies if the matter has been assessed as part of a planning permit application. |
| 404 | 69 | What is an existing building? Defines ‘existing building’ for the purposes of pt 6, to clarify how the reference is to be treated in relation to an adjoining property. |  | ✓ |  |  |  | This Regulation ensures consistency in the interpretation of the term ‘existing building’ for the purposes of this part of the Regulations. |
| 405 | 70 | What is a single dwelling? Clarifies that the Ministerial Guidelines only apply to a single dwelling and structures associated with those dwellings, such as sheds and carports. |  | ✓ |  |  |  | This Regulation ensures consistency in interpretation between the Act and Regulations. |
| 406 | 71 | Approved building envelope design overrides siting requirement Clarifies when pt 6 applies to proposed buildings, where a building envelope applies to site and deals with the same matter. |  |  | ✓ |  |  | This Regulation manages inconsistencies between pt 6 and an approved building envelope. It ensures effective integration with the planning system and has been updated to include definitions. |

#### Div 2—Single Class 1 buildings and associated Class 10 buildings

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 407 | 72 | Application of division Requires single dwellings and certain structures associated with those dwellings, such as sheds and carports to comply with proposed regs 73–86. |  | ✓ |  |  |  | This Regulation specifies the types of buildings that must comply with these requirements. |
| 408 | 73 | Maximum street setback Restricts how far back the building can be from the street. |  | ✓ |  |  |  | This Regulation intends to manage inappropriate subdivision, which is likely to be encouraged if owners are permitted to develop at the rear of a block. |
| 409 | 74 | Minimum street setbacks Restricts the distance that a building can be positioned in relation to a front or side street. |  | ✓ |  |  |  | This Regulation reflects Standard A3 (cl 54.03-1 of the Victoria Planning Provisions). It ensures that residential developments make efficient use of the site and that street character be respected. |
| 410 | 75 | Building height Restricts how high the building can be. |  | ✓ |  |  |  | This Regulation reflects Standard A4 (cl 54.03-2 of the Victoria Planning Provisions). It protects the amenity of existing buildings and reduces bulk and high dwellings. |
| 411 | 76 | Site coverage Restricts how much of the property can be covered by the building. |  | ✓ |  |  |  | This Regulation reflects Standard A5 (cl 54.03-3 of the Victoria Planning Provisions). It ensures adequate outdoor space is provided for the amenity of occupiers to a dwelling. |
| 412 | 77 | Permeability Specifies how much of the property must have absorbent surfaces. |  | ✓ |  |  |  | This Regulation reflects Standard A6 (cl 54.03-4 of the Victoria Planning Provisions). It manages the amount of stormwater that runs off a property. |
| 413 | 78 | Car parking Specifies car parking requirements. |  | ✓ |  |  |  | This Regulation reflects cl 52.06 of the Victoria Planning Provisions. It ensures that new dwellings have adequate onsite car parking. |
| 414 | 79 | Side and rear setbacks Restricts the distance a building can be positioned in relation to the side and rear boundary. |  |  |  | ✓ |  | This Regulation reflects Standard A10 (cl 54.04-1 of the Victoria Planning Provisions). It ensures that there is adequate separation between buildings on adjoining properties and includes eaves, fascia and gutters as allowable encroachments into the setback. |
| 415 | 80 | Walls and carports on boundaries Restricts the height and length of walls or carports along the boundary. |  | ✓ |  |  |  | This Regulation reflects Standard A11 (cl 54.04-2 of the Victoria Planning Provisions) and protects the amenity of adjoining allotments. |
| 416 | 81 | Daylight to existing habitable room windows Restricts how close a building can be positioned in relation to windows of an existing dwelling. |  | ✓ |  |  |  | This Regulation reflects Standard A12 (cl 54.04-3 of the Victoria Planning Provisions). It ensures adequate daylight is provided to existing habitable room windows on adjoining allotments. |
| 417 | 82 | Solar access to existing north-facing windows Restricts how close a building can be constructed in relation to an existing dwellings north-facing windows. |  |  |  | ✓ |  | This Regulation reflects Standard A13 (cl 54.04-4 of the Victoria Planning Provisions). It protects the energy efficiency of existing dwellings which receive passive solar heating. Aspects of this Regulation are transferred to a definition ('north-facing windows') and does not apply to existing windows, which will be situated above the eaves of the proposed dwelling. |
| 418 | 83 | Overshadowing of recreational private open space Restricts the construction of a building if it reduces sunlight to the outdoor space of an existing dwelling. |  |  | ✓ |  |  | This Regulation reflects Standard A14 (cl 54.04-5 of the Victoria Planning Provisions). It protects existing recreation private space of the adjoining property. Aspects of the Regulation have been transferred to the definition of 'recreational private open space'. |
| 419 | 84 | Overlooking Restricts the construction of a building if it reduces the privacy of an existing dwelling. |  | ✓ |  |  |  | This Regulation reflects Standard A15 (cl 54.04-6 of the Victoria Planning Provisions). It protects existing habitable rooms and secluded private open space from overlooking. |
| 420 | 85 | Daylight to habitable room windows Specifies the minimum design requirements for habitable room windows. |  | ✓ |  |  |  | This Regulation reflects Standard A16 (cl 54.05-1 of the Victoria Planning Provisions). It ensures that all windows of habitable rooms receive adequate light. |
| 421 | 86 | Private open space Specifies the required area of private open space that is required on an allotment for residential dwellings. |  | ✓ |  |  |  | This Regulation reflects Standard A17 (cl 54.05-2 of the Victoria Planning Provisions). It ensures that residents have access to adequate private open space for their reasonable recreation and service needs. |

#### Div 3—Siting of Class 10a buildings

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 422 | 87 | Sitting of Class 10a buildings Prevents sheds and the like from being constructed on an allotment, unless it is appurtenant to another building on the same allotment. |  |  |  | ✓ |  | This Regulation ensures that nonhabitable buildings do not have a negative amenity impact on neighbouring properties. A Class 10a building (with a floor area less than 10 m²) to be constructed on an otherwise vacant allotment is consistent with an existing exemption in proposed sch 4 (see item 1). |

#### Div 4—Class 10b structures

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 423 | 88 | Application of division Requires fences and other Class 10b structures to comply with regs 424–431. |  | ✓ |  |  |  | This Regulation specifies the types of structures that must comply with these requirements. It ensures nonhabitable structures do not have adverse siting and amenity impacts. |
| 424 | 89 | Front fence height Restricts how high a fence can be that within 3 m of the street alignment. |  | ✓ |  |  |  | This Regulation reflects Standard A20 (cl 54.06-2 of the Victoria Planning Provisions). It encourages design detail that reflects the existing or preferred neighbourhood character. |
| 425 | 90 | Fence setbacks from side and rear boundaries Same as reg 414 but relates to fences. |  |  |  | ✓ |  | Refer reasoning in relation to proposed reg 79. |
| 426 | 91 | Fences on or within 150 mm of side or rear boundaries Same as reg 415 but relates to fences. |  | ✓ |  |  |  | Refer reasoning in relation to proposed reg 80. |
| 427(1) | 92 | Fences on intersecting street alignments Restricts how high a fence can be within 9 m of an intersection. |  | ✓ |  |  |  | Mitigates risk. Fences on intersections can be safety risk to motorists if they are blocking their line of sight. There are structural changes to how this Regulation has been drafted. |
| 427(2) | 93 | Fences must not include barbed wire Restricts the installation of fences with sharp protrusions where the fence is adjacent to streets or public spaces. |  |  | ✓ |  |  | Mitigates risk. Sharp protrusions can be a public hazard. The Regulation has been updated to clarify that barbed wire fencing can be allowed for agricultural purposes. |
| 428 | 94 | Fences and daylight to windows in existing dwelling Same as reg 416 but relates to fences. |  | ✓ |  |  |  | Refer reasoning in relation to proposed reg 81. |
| 429 | 95 | Fences and solar access to existing north-facing habitable room windows Same as reg 417 but relates to fences. |  |  |  | ✓ |  | Refer reasoning in relation to proposed reg 82. |
| 430 | 96 | Fences and overshadowing of recreational private open space Same as reg 418 but relates to fences. |  |  | ✓ |  |  | Refer reasoning in relation to proposed reg 83. |
| 431 | 97 | Masts, poles etc. Prohibits the construction of tall Class 10b structures. |  | ✓ |  |  |  | Ensures that nonhabitable structures do not have a negative amenity impact on neighbouring properties. |

### Pt 7—Projections beyond street alignment

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 504 | 98 | Projections beyond the street alignment Prohibits any part of a building from extending out beyond an allotment boundary over a footpath or street. |  | ✓ |  |  |  | This Regulation is required to protects the health and safety of the public by ensuring buildings do not impact on the safe movement of pedestrians and/or vehicles on footpaths and streets. |
| 505(1)(b) 505(1)(d) 505(1)(e) 505(2) | 99 | Architectural features – narrow street Limits when an architectural feature can extend out from a building over a narrow street. |  |  | ✓ |  |  | This Regulation is required to ensure suitable clearances for architectural features are provided to allow for the safe passage of pedestrian and vehicular traffic For clarity, this requirement has been made the subject of its own Regulation and the term ‘narrow street’ is now defined in the definitions clause of the Regulations. |
| 505(1)(b) 505(1)(d) 505(1)(e) | 100 | Architectural features – medium street Limits when an architectural feature can extend out from a building over a medium street. |  |  | ✓ |  |  | This Regulation is required to ensure suitable clearances for architectural features are provided for the safe passage of pedestrian and vehicular traffic. For clarity, this requirement has been made the subject of its own Regulation and the term ‘medium street’ is now defined in the definitions clause of the Regulations. |
| 505(1)(c) 505(1)(d) 505(1)(e) | 101 | Architectural – wide street Limits when an architectural feature can extend out from a building over a wide street. |  |  | ✓ |  |  | This Regulation is required to ensure suitable clearances for architectural features are provided for the safe passage of pedestrian and vehicular traffic. For clarity, this requirement has been made the subject of its own Regulation and the term ‘wide street’ is now defined in the definitions clause of the Regulations. |
| 506 | 102 | Windows and balconies Limits when a balcony or window can extend out from a building over a street or footpath. |  |  | ✓ |  |  | This Regulation is required to ensure suitable clearances for windows and balconies are provided for the safe passage of pedestrian and vehicular traffic. This Regulation has been updated to specify how permissible clearances will apply to narrow, medium and wide streets. |
| 507 | 103 | Verandahs Limits when verandas can extend out from a building over a street or footpath. |  | ✓ |  |  |  | This Regulation is required to ensure suitable clearances for verandahs are provided for the safe passage of pedestrian and vehicular traffic. |
| 508 | 104 | Sunblinds and awnings Limits when sunblinds and awnings can extend out from the building over a street or footpath. |  |  |  |  | ✓ | This Regulation is required to ensure suitable clearances for sunblinds and awnings are provided for the safe passage of pedestrian and vehicular traffic. The clearance for vehicle traffic is new. |
| 509 | 105 | Service pipes and rainwater heads Restricts when service pipes and rainwater heads can extend out from a building over a street or footpath. |  | ✓ |  |  |  | This Regulation is required to ensure suitable clearances are provided for protecting pedestrian traffic. |
| 510 | 106 | Window shutters Limits when window shutters can extend out from a building over a street or footpath. |  | ✓ |  |  |  | This Regulation is required to ensure suitable clearance is provided for protecting pedestrian traffic. |
| 511 | 107 | Signs Limits when signs or other Class 10b structures can extend out form a building over a street or footpath. |  | ✓ |  |  |  | This Regulation is required to ensure suitable clearance is provided for protecting pedestrian traffic. |
| 512 | 108 | Service cabinet doors Limits when service cabinet doors, when open, can extend from a building over a street or footpath. |  | ✓ |  |  |  | This Regulation is required to ensure suitable clearance is provided for protecting pedestrian traffic. |
| 513 | 109 | Report and consent required Requires a report and consent of the relevant council for a projection which is proposed to extend at a vertical and/or horizontal clearance which is different from a clearance specified in pt 7 of the Regulations. |  | ✓ |  |  |  | This Regulation is required to provides flexibility for building permit applicants by allowing the relevant council to authorise a noncompliant projection if it will not have an adverse impact on public safety. |
| 514 | 110 | Footings adjoining boundaries are permissible Allows a footing to support party wall. |  | ✓ |  |  |  | This Regulation provides generic clearances for footings proposed to extend beyond the street alignment. |

### Pt 8—Protection of adjoining property and the public

#### Div 1—Protection of adjoining property by means of protection work

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 602 | 111 | Required protection work Requires the RBS to determine if building work gives rise to the need for protection work. It also provides for how this determination should be documented and the owner notified of the outcome of the determination. |  |  |  |  | ✓ | This Regulation is required to triggers the application of pt 7 of the Act to building work which gives rise to the need for protection work. This requirement has been strengthened to require the RBS to document and record a determination to require (or not the require) protection work in the prescribed form. |
| 603 | 112 | Matters RBS must consider when determining if protection work required Specifies matters which the RBS must consider when making a determination as to whether or not building work gives rise to the need for protection work. |  |  |  |  | ✓ | This Regulation is required to set out the mandatory minimum criteria which the RBS must take into account when making a determination in relation to whether or not building work gives rise to the need for protection work. This is a new Regulation to support consistent decision-making between building surveyors, in respect of the need for protection work. |
| 602(2) 602(3) | 113 | Protection work notice Specifies the form to be used for a protection work notice required by s 84 of the Act and also the information which must accompany that form when served. |  |  |  |  | ✓ | This Regulation is required to give effect to s 84 of the Act. Amended to strengthen the operation of the Regulation to address information asymmetry for the benefit of the adjoining owner. |
| 602(4) 602(5) | 114 | Protection work response notice Specifies the form a response notice of an adjoining owner under s 85 of the Act must take and also the information which must be contained in that notice. |  | ✓ |  |  |  | This Regulation is required to give effect to s 85 of the Act. |
| 602(6) | 115 | Notice of RBS determination Specifies the form the RBS must use to make a determination under s 87 of the Act and also the information which the RBS must include in the determination. |  |  | ✓ |  |  | This Regulation is required to give effect to s 87 of the Act. It clarifies information which must be included in the determination and to require the determination to be documented and recorded in a prescribed form. |

#### Div 2—Protection of the public

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 604 | 116 | Protection of the public Requires the RBS to consider if proposed building work will pose a risk to the safety of the public. If the RBS considers that building work will pose such a risk, the RBS must require safety measures to be put in place to guard against that risk both before and during the building work which gives rise to that risk. |  | ✓ |  |  |  | This Regulation is required to ensure the RBS manages health and safety risks to the public before and during building work. |

#### Div 3—Demolition

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 607 | 117 | Demolition Imposes minimum required safety standards for demolition. It also grants the RBS the power to exempt minor demolition work from these requirements in appropriate circumstances. |  | ✓ |  |  |  | This Regulation is required to mitigate health and safety risks associated with demolition, including the removal of hazardous waste. |

#### Div 4—Excavations and retaining walls

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 605 | 118 | Excavations Imposes safety and building standards which are specific to excavations. |  | ✓ |  |  |  | This Regulation is required to mitigate health and safety risks associated with excavations and to ensure foundation soil is not subject to significant moisture. |
| 606 | 119 | Retaining walls Empowers the MBS to require retaining walls or other means of maintaining the stability of the soil on an owner’s allotment or an adjoining property in circumstances where any excavation or soil filling activity may adversely effect the stability of the same. |  | ✓ |  |  |  | This Regulation is required to ensure power exists for the MBS to require an owner to provide retaining walls or other means of maintaining stability of soil on their own allotment or an adjoining allotment to protect against potential collapse or other adverse consequences which arise as a result of excavation or soil infilling work on the owner’s property. |

### Pt 9—Building work

#### Div 1—Methods of assessment of compliance

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 601 | 120 | Testing of materials Enables the RBS to require testing of any material used in building work in respect of which a building permit has been issued, and prohibit the use of any material if, after testing, is found to be noncompliant or unsuitable or unfit for its intended use. |  | ✓ |  |  |  | This Regulation is required to assist the RBS to monitor and enforce building standards in respect of building work which they have approved under a building permit. The objective of this Regulation is to reduce risks associated with the use of noncompliant building materials by providing additional powers to the RBS to verify that building materials meet relevant building standards. |
| 113 | 121 | Fire-performance requirements—performance solutions Limits the circumstances in which the RBS is empowered to determine that a performance solution complies with a fire-performance requirement of the BCA. |  |  |  |  | ✓ | This Regulation is required to reduce health and safety risks associated with building fires by requiring additional expert oversight of approvals of fire-performance solutions owing to the complex nature of fire-safety measures. This Regulation remakes current reg 113 with amendments to clarify options to meet the additional layer of fire expertise required to approve fire-performance solutions. |
| 1505 | 122 | Certificates of compliance—proposed building work Specifies the categories and classes of building practitioner who may give a certificate of compliance in respect of certain types of proposed building work. |  | ✓ |  |  |  | This Regulation is required to prescribe information to give effect to s 238(1)(a) of the Act. |
| 1506 | 123 | Certificates of compliance—building work Specifies the categories and classes of building practitioner who may give a certificate of compliance in respect of certain types of building work. |  | ✓ |  |  |  | This Regulation is required to prescribe information to give effect to s 238(1)(b) of the Act. |
| 111 | 124 | Assessment Method—Evidence of suitability—certificate from professional engineer or other qualified person This Regulation aligns the BCA with the Act with the 2017 Regulations by specifying who has appropriate expertise to determine a building solution complies with the performance requirements of the BCA. |  |  | ✓ |  |  | This Regulation provides that this evidence of suitability in the form of a certificate from a professional engineer or other appropriately qualified person must be provided as a certificate of a prescribed building practitioner under s 238 of the Act complying with current regs 122 or 123 (as the case requires). |
| 1507 | 125 | Forms of certificates of compliance Provides that a building practitioner may issue a certificate of compliance in the relevant form approved by the VBA. |  | ✓ |  |  |  | This Regulation is required to authorise the VBA to produce a form for certificates of compliance. |

#### Div 2—Special provisions

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 612 | 126 | Record of pile-driving Requires a person undertaking pile-driving operations to keep records of those operations and provide those records to the RBS within 28 days after completion of the relevant pile-driving operations. |  | ✓ |  |  |  | This Regulation is required to ensures that records are kept as evidence that compliant pile-driving operations have been carried out to ensure the adequate load-bearing capacity of the foundations of a building. |
| 613 | 127 | Branding of timber Requires every piece of timber used for structural building work to be stress graded and marked accordingly. |  | ✓ |  |  |  | This Regulation operates in addition to requirements in the BCA and has been revised to updated technical standards. |
| 309 | 128 | Requirements for permits involving fire-safety matters Specifies when the report and consent of a chief officer is required in relation to a building permit application. |  |  | ✓ |  |  | Mitigates risks for firefighters caused by information asymmetry. Updated to remove fire reels and proscenium curtain drencher systems (not commonly required) and to add fire extinguishers (are more commonly required), and the emergency vehicle access provision is redrafted to make it easier to understand. |
| 310 | 129 | Report and consent for building over easements Specifies when the report and consent of a service authority is required in relation to a building permit application. |  |  | ✓ |  |  | This Regulation is required to align the building regulatory framework with the property rights of service authorities. |
| 311 | 130 | Report concerning need for electricity substations Enables the RBS to require a report from the relevant electricity supply authority in relation to a building permit application for a building other than a Class 1 or Class 10 building. |  | ✓ |  |  |  | This Regulation is required to better align the building regulatory framework with the infrastructure needs of electricity supply authorities by allowing the RBS to take into inquire whether there will be adequate electricity infrastructure for any applicable proposed development. |
| 801 | 131 | Report and consent for septic tank systems Requires the report and consent of the relevant council in a relation to an application for a building permit that requires a septic tank, if approval for work in relation to a septic tank has not been given under the Environment Protection Act 1970. |  | ✓ |  |  |  | This Regulation is required to mitigates health risks associated with incorrectly installed septic tanks and provides a checkpoint that the septic tank has a permit under s 53L of the Environment Protection Act 1970. |
| 610 | 132 | Storm water drainage Requires specific additional approval requirements in relation to stormwater drainage systems, including a requirement to obtain a report from the relevant council in relation to how a proposed point of discharge aligns with council infrastructure. |  |  |  |  | ✓ | This Regulation is required to ensure the proper management of stormwater, including to reduces the risk of flooding arising as a result of ineffective stormwater drainage systems. Minor amendment is proposed to current reg 612 to ensure that the design of stormwater drainage system from the property is only approved if the point of discharge is consistent with the report of the relevant council. |
| 515 | 133 | Building above or below certain public facilities Requires the report and consent of the relevant council to the construction of a building which is proposed above or below public infrastructure. |  | ✓ |  |  |  | This Regulation is required to align public and private infrastructure requirements to mitigate risks to public health and safety. |

### Pt 10—Building work—safety requirements

#### Div 1—Requirements for barriers for swimming pools and spas until 1 October 2020

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 701 | 134 | Definition of owner Defines 'owner' in relation to a swimming pool and/or spa. |  | ✓ |  |  |  | This Regulation reduces inconsistency in interpretation. The owner of a swimming pool and/or spa is best placed to manage the risks associated with the use of those structures. |
| 702 | 135 | Application and revocation of this division Specifies which swimming pools and spas are required to comply with Div 1 of the proposed Regulations. |  |  |  |  | ✓ | This Regulation is required to revoke the current Regulations to enable proposed Regulations 139–141 to come into effect in 2020. |
| 703 | 136 | Requirement for barriers Requires specific safety barriers and gates for a swimming pool and/or spa. |  | ✓ |  |  |  | This Regulation mitigates risks to health and safety by providing for a barrier to prevent a child from being able to access the pool area. |
| 704 | 137 | Requirements for doors or gates Requires all doors and gates, which form part of a swimming pool and/or spa safety barrier, must be self-closing. |  | ✓ |  |  |  | This Regulation mitigates risks to health and safety by preventing a door or gate from being left open by accident, if left open a child will be able to access the pool area. |
| 705 | 138 | Requirements for windows in walls used as barriers Establishes the requirements for windows in walls which form part of a safety barrier. |  | ✓ |  |  |  | This Regulation mitigates risks to public health and safety by requiring additional safety features to be installed on windows, which a child could climb through to access the pool area. |

#### Div 2—Requirements for barriers for swimming pools and spas from 1 October 2020

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 701 | 139 | Definition of owner Defines 'owner' in relation to a swimming pool and/or spa. |  | ✓ |  |  |  | This Regulation promotes consistency in interpretation. The owner of a swimming pool and/or spa is best placed to manage the risks associated with the use of those structures. |
| n/a | NEW 140 | Application of this division Specifies which swimming pools and spas are required to comply with div 2 of the proposed Regulations. |  |  |  |  | ✓ | Refer also to proposed reg 135. This Regulation will apply standards retrospectively to existing swimming pools and spas constructed before 1 May 2010, because they will not have a barrier that meets current safety standards. |
| n/a | NEW 141 | Swimming pool and spa barriers Requires safety barriers to be upgraded to the current standards, which currently apply to all new swimming pools and spas. |  |  |  |  | ✓ | Refer also to proposed reg 135. This Regulation mitigates risks to health and safety by requiring the upgrading of safety barriers to current standards, which is the installation of a four-sided isolation barrier around all existing swimming pools and/or spas. |

#### Div 3—Maintenance and operation of barriers for swimming pools and spas

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1219 | 142 | Application of this division This Regulation applies to swimming pools and/or spas that belong to classes 1,2,3,4 or 10 buildings will be affected by the requirements under div 2 of the proposed Regulations. |  | ✓ |  |  |  | This Regulation specifies the class of building that is required to comply with the proposed Regulations in this division. |
| n/a | NEW 143 | Swimming pool and spa barrier maintenance Requires the owner to maintain the pool and/or spa safety barrier. |  |  |  |  | ✓ | This Regulation mitigates risks of safety barriers becoming ineffectual due to poor maintenance. This obligation is similar to proposed reg 144, however, there was previously no obligation imposed on the owner. |
| 1220(1) | 144 | Swimming pool and spa barrier operation Requires an occupier to ensure that the pool and/or spa safety barrier is operating effectively. |  |  |  |  | ✓ | This Regulation mitigates safety risks by imposing an obligation on the occupier to ensure that the safety barrier is in working order. The obligation is imposed specifically on the occupier. |
| 1220(2) 1220(3) | 145 | Swimming pool and spa barrier gate must remain closed Establishes the requirement that any gate or door, which forms part of a safety barrier, must remain closed except when a person is entering or leaving the pool or spa area. |  |  |  |  | ✓ | This Regulation mitigates safety risks by imposing an obligation on the occupier to ensure that the any gates or doors remain closed. The broader application to any person is removed. This strengthens the offence because it is straightforward to determine liability because it now falls on the occupier. It is too difficult to determine liability if there are multiple people using the gate or door on a day to day basis. |

#### Div 4—Fire safety in certain existing residential buildings

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 706 | 146 | Definition of owner Defines an owner. |  | ✓ |  |  |  | This Regulation promotes consistency in interpretation. |
| 707 709 | 147 | Automatic smoke detection and alarm systems for residential care buildings Requires all residential care buildings to have an automatic smoke detection and alarm system that complies with specification E2.2a of the BCA Volume One installed. |  |  | ✓ |  |  | This Regulation protects health and safety of occupants, who may be vulnerable to fire risks, by ensuring older buildings have early detection and warning systems installed to enable escape in the event of a fire. Combines current regs 707 and 709. The proposed Regulation is updated as a current requirement as all buildings should have been brought into compliance with this Regulation. |
| 708 | 148 | Residential care buildings—automatic fire sprinkler systems Requires fire sprinkler systems to be installed in residential care buildings. |  |  | ✓ |  |  | This Regulation protects health and safety of occupants, who may have difficulty moving quickly in a fire, by ensuring older buildings have adequate fire-suppression systems installed. The proposed Regulation is updated as a current requirement as all buildings should have been brought into compliance with this Regulation. |
| 710 | 149 | Shared accommodation buildings—automatic sprinkler systems Requires fire sprinkler systems to be installed in all shared accommodation buildings. |  |  | ✓ |  |  | This Regulation protects health and safety of occupiers using of temporary accommodation by requiring older buildings to have adequate fire-suppression systems installed. The proposed Regulation is updated as a current requirement as all buildings should have been brought into compliance with this Regulation. |

### Pt 11—Designation of special areas for building work

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 807(1) 807(6) | 150 | Mapping of designated special areas Requires council to produce maps of designated special areas. The term ‘designated special area’ is defined in the new definitions clause of the proposed 2017 Regulations. |  | ✓ |  |  |  | This Regulation is required to ensure councils keep up-to-date records to identify and map designated special areas for the benefit of the public and the operation of the 2017 Regulations. |
| 8O7(2) - 807(5) | 151 | Alteration to and creation of new designated special area Specifies the administrative obligations that the council must discharge in meeting their obligation to map special designated areas. |  | ✓ |  |  |  | This Regulation addresses information asymmetry by ensuring information about the location of special designated areas is accessible. |
| 803(1) | 152 | Termite-risk areas Enables council to declare termite areas. |  | ✓ |  |  |  | This Regulation addresses information asymmetry by enabling council to declare termite areas. |
| 803(2) | 153 | Construction in termite-risk areas Specifies when a building is required to comply with the termite management requirements in the BCA. |  | ✓ |  |  |  | This Regulation modifies the application of the BCA (reduces the compliance burden). |
| 805 | 154 | Significant snowfall areas Enables council to declare alpine areas. |  | ✓ |  |  |  | This Regulation protects health and safety by ensuring buildings are safe during significant snow fall. Clarifies application of BCA. |
| 802(1) 802(3)–802(8) | 155 | Report and consent for building in areas liable to flooding Requires the report and consent to be obtained from the relevant council if the building site is in an area liable to flooding. |  |  |  | ✓ |  | This Regulation reduces impacts of flooding on buildings and building occupants. The requirement has been adjusted to apply only to residential use buildings. Previously it also applied to Class 5, 6, 7, 8 or Class 9b. These classes are now exempt. Land liable to flooding has been moved to the definitions Regulation. |
| 806 | 156 | Report and consent for construction on designated land or designated works Requires the report and consent of the relevant council if the construction of a building is proposed on designated land or works. |  | ✓ |  |  |  | This Regulation mitigates risks to public infrastructure by providing a checkpoint to ensure that approval is obtained from the relevant authorities. Designated land or works moved to definitions Regulation. |

### Pt 12—Bushfire safety

#### Div 1—Construction in designated bushfire prone areas

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 810 | 157 | Designated bushfire-prone areas Enables the Minister to declare designated bushfire-prone areas and to map those areas. |  | ✓ |  |  |  | This Regulation addresses information asymmetry by identifying the areas that are at risk of bushfires. |
| 811 | 158 | RBS must accept bushfire attack level (BAL) in planning scheme or site assessment for planning permit Requires that a BAL that has been determined under the planning system must be applied to a building permit. The BCA is modified to increase the minimum standards for residential buildings. |  | ✓ |  |  |  | This Regulation reduces duplication with planning system requirements and the construction, in accordance with the BAL rating is aimed at protecting health and safety of occupants of residential properties. |
| 812 | 159 | RBS must accept BAL of 12-5 Requires residential buildings and buildings occupied by young children and the elderly to be constructed to a minimum of 12.5 BAL in a bushfire-prone area. |  | ✓ |  |  |  | This Regulation mitigates health and safety risks for occupants of residential buildings, or for occupants who are vulnerable to bushfire (residents and occupants who can not readily evacuate buildings (due to health or age). |
| 114 | 160 | Specific use bushfire protected buildings—Construction requirements Defines a specific use bushfire protected building. |  | ✓ |  |  |  | This Regulation promotes consistent interpretation. |

#### Div 2—Requirements for buildings replacing buildings destroyed in the 2009 bushfires

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 808(1) 809(1) | 161 | Application of this division Specifies the properties that are required to comply with this division. |  | ✓ |  |  |  | This Regulation reduces burden for properties that burned down in bushfires in early 2009 by allowing the properties to be rebuilt without a planning permit. |
| 808(2)–(8) | 162 | Static water supply for firefighting purposes Specifies the requirements for a static water supply to be provided at the property. |  | ✓ |  |  |  | This Regulation reduces the risks of bushfires by ensuring firefighters have access to water (controls usually imposed by the planning system). |
| 809 | 163 | Access for emergency vehicles Specifies minimum access requirements for driveways to accommodate emergency vehicles. |  | ✓ |  |  |  | This Regulation protects health and safety by ensuring that emergency vehicles can access properties (controls usually imposed by the planning system). |
| 309A | 164 | Report and consent required for noncomplying static water supply and access Requires the report and consent of a chief officer to be obtained if requirements can not be met. |  | ✓ |  |  |  | This Regulation ensures appropriate oversight of bushfire safety if properties do not comply with the requirements of this division. Regulation is more streamlined. |

#### Div 3—Private bushfire shelters

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1221 | 165 | Application of this division Specifies that private bushfire shelters that are associated with a residential property are required to comply with this division. |  | ✓ |  |  |  | This Regulation promotes consistent interpretation. |
| 115 | 166 | Private bushfire shelter construction requirements This Regulation modifies the BCA by changing Class 1a to Class 1. |  | ✓ |  |  |  | This Regulation protects health and safety by ensuring that private bushfire shelters attached to bed and breakfast or hostels are also required to comply with the BCA. |
| 113A | 167 | Private bushfire shelter performance requirements—performance solutions This Regulation restricts who can determine that the design complies with a bushfire performance requirement. |  | ✓ |  |  |  | This Regulation mitigates health and safety risks by ensuring alternative solutions designed to meet the bushfire performance requirements of the BCA are subject to expert judgement. |
| 1222 | 168 | Private bushfire shelter maintenance and operation Requires a private bushfire shelter to be maintained, including the pathway from the residence to the shelter. |  | ✓ |  |  |  | This Regulation protects health and safety by requiring the shelter to be maintained to prevent it from deteriorating over time, or being inaccessible or blocked, or being used as a storage unit. |

### Pt 13—Inspections notices and orders

#### Div 1—Inspections and directions

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 901(1) | 169 | Prescribed mandatory notification stages for construction or alteration of building Specifies when the builder must notify the RBS in relation to a completed stage of building work. |  |  |  |  | ✓ | This Regulation gives effect to s 3(1) of the Act. Three additional mandatory notification stages are prescribed to manage risks associated with noncompliant work. |
| 901(1) | 170 | Prescribed mandatory notification stages for demolition or removal of building Specifies when the builder must notify the RBS in relation to a completed stage of building work associated with the demolition or removal of a building. |  |  | ✓ |  |  | This Regulation gives effect to s 3(1) of the Act. Specified separately for this type of building work to improve compliance. |
| 901(1) | 171 | Prescribed mandatory notification stages for construction of swimming pool Specifies when the builder must notify the RBS in relation to a completed stage of building work associated with the construction of a swimming pool. |  |  | ✓ |  |  | This Regulation gives effect to s 3(1) of the Act. Specified separately for this type of building work to improve compliance. |
| 901(2) | 172 | Variation of mandatory notification stages Enables the RBS to vary a mandatory notification stage if it is not relevant to the building work, demolition or removal of a building. |  |  | ✓ |  |  | This Regulation reduces delay costs associated with unnecessary inspections. |
| 901(3) | 173 | Building permit must specific mandatory notification stages Requires the RBS to specify all mandatory stages on the building permit issued for that building work. |  |  | ✓ |  |  | This Regulation addresses information asymmetry because the mandatory notification stages for each building permit are specified. This Regulation has been updated to take into account the remaking of current reg 901(1), which has been split into three Regulations to refer to specific types of building work. |
| 902 | 174 | RBS to keep and make available directions to fix building work Specifies the administrative obligations of the RBS in relation issuing direction under pt 4 div 2 of the Act. |  | ✓ |  |  |  | This Regulation addresses information asymmetry. Ensures that adequate information is kept in relation to directions issued by RBSs to persons in charge of building work. |
| 902A | 175 | Person to whom authorised person must give documents under s 37J of the Act Specifies the RBS as the person whom an authorised person must give documents to in relation to a written direction to fix building work. |  | ✓ |  |  |  | This Regulation gives effect to s 37J of the Act. |

#### Div 2—Notices and orders

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 903 | 176 | Emergency orders Specifies details that an emergency order must contain. |  |  | ✓ |  |  | This Regulation gives effect to s 103(3) of the Act. Additional information required to be recorded if another person conducts the inspection on the behalf of a RBS. |
| n/a | NEW 177 | Form of emergency orders Provides a nonmandatory form to be used when issuing an emergency order. |  |  |  |  | ✓ | This Regulation aims to improve the issuing of an emergency order because it will ensure that it contains all mandatory information and is enforceable at law. |
| 904 | 178 | Building notices Specifies details that a building notice must contain. |  |  | ✓ |  |  | This Regulation gives effect to s 103(3) of the Act. Additional information required to be recorded if another person conducts the inspection on the behalf of a RBS. |
| n/a | NEW 179 | Form of building notice Provides a nonmandatory form to be used when issuing a building notice. |  |  |  |  | ✓ | This Regulation aims to improve the issuing of a building order because it will ensure that it contains all mandatory information and is enforceable at law. An invalid building notice will affect the legality of a subsequent building order issued and served under s 111 of the Act. |
| 905 | 180 | Building orders Specifies details that a building order must contain. |  |  | ✓ |  |  | This Regulation gives effect to s 114(1)(c) of the Act. Additional information required to be recorded if another person conducts the inspection on the behalf of a RBS. |
| n/a | NEW 181 | Form of building order Provides a nonmandatory form to be used when issuing a building notice. |  |  |  |  | ✓ | This Regulation aims to improve the issuing of a building notice because it will ensure that it contains all mandatory information and is enforceable at law. |

### Pt 14—Occupancy Permits and Certificates of Final Inspection

#### Div 1—General

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1001(1) 1801 | 182 | Application of part Specifies the buildings that must comply with this part of the Regulations. |  | ✓ |  |  |  | This Regulation specifies the buildings that must comply with pt. 14 of the proposed Regulations. |
| 1001(2) | 183 | Exemptions from occupancy permit Provides exemptions from the requirement to have an occupancy permit. See also sch 3 to the proposed Regulations. |  | ✓ |  |  |  | This Regulation specifies the exemptions that apply in relation to the buildings that are required to comply with pt. 14 of the proposed Regulations. Reduces burden for low-risk buildings and building work. |

#### Div 2—Applications for occupancy permit

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1002 | 184 | Application for occupancy permit Requires an application for an occupancy permit to be in a prescribed form. |  |  |  |  | ✓ | This Regulation will improve administrative efficiency and reduces delays. Requires additional information to be submitted with an application to ensure the RBS has all the correct details to make a decision about whether to authorise occupation of a building. |
| 1003 | 185 | Reporting authorities for occupancy permit Specifies who is a reporting authority in relation to an application for an occupancy permit. |  | ✓ |  |  |  | This Regulation mitigates risks associated with inadequate compliance with the reporting authority recommendations. |
| 1004 | Multiple | Has been replaced by multiple Regulations. Discussed below in relation to proposed Regulations 186, 187 and 188. These proposed Regulations also replace sch 6 of the current Regulations. |  |  |  |  |  |  |
| Sch 6 Item 1 | 186 | Time limits for application to be given to reporting authority Specifies the time period when a copy of an occupancy permit application must be provided to each relevant reporting authority within. |  | ✓ |  |  |  | This Regulation gives effect to cl 4(2) of sch 2 to the Act. Regulation structure has been changed into a narrative instead of a Table format. |
| Sch 6 Item 2 | 187 | Time limits relating to the report or consent of reporting authorities Specifies the time period the relevant reporting authority must respond after receiving the occupancy permit application. |  | ✓ |  |  |  | This Regulation gives effect to cls 6(1), (2) and (3) of the Act. Regulation structure has been changed into a narrative instead of a Table format. |
| Sch 6 Item 3 | 188 | Time within which RBS must decide application for occupancy permit Specifies the time period the RBS must decide on an application for a building permit after receiving the report or consent from. |  | ✓ |  |  |  | This Regulation gives effect to ss 43 and 56 of the Act. Regulation structure has been changed into a narrative instead of a Table format. |
| n/a | NEW 189 | Time within which RBS must decide application to amend occupancy permit Specifies the time period the RBS must decide to amend an occupancy permit after receiving an application. |  |  |  |  | ✓ | This Regulation supports the operation of s 70(1)(a) of the Act. |

#### Div 3—Occupancy permits

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1005 | 190 | Form of occupancy permit Requires an occupancy permit to be issued in the prescribed form. |  |  | ✓ |  |  | This Regulation addresses information asymmetry. Facilitates compliance, monitoring and enforcement. Ensures that there is adequate information to check that occupation has been duly authorised. Additional information required to be set out in the occupancy permit in relation to any performance solution being used and describing the conditions in relation to ESMs (see proposed form 18). |
| n/a | NEW 191 | RBS to provide copy of permit Requires the RBS to provide the applicant with a copy of the occupancy permit within seven days. |  |  |  |  | ✓ | This Regulation reduces delays in the provision of the occupancy permit and enables the commencement of the relevant appeal period, for example if the owner wants to appeal a condition on the occupancy permit. Seven days is consistent with requirements in relation to a building permit. |
| 1203 | 192 | Occupancy permit must specify ESMs and related requirements Requires an occupancy permit to specify the maintenance requirements of all ESMs in the building or place of public entertainment. |  |  | ✓ |  |  | This Regulation addresses information asymmetry and mitigates health and safety issues that arise if an ESM is not maintained. It has been updated to clarify how ESM conditions should be recorded in relation to ESM. The occupancy permit is the first point of reference for ESM maintenance obligations for a building owner. |
| n/a | NEW 193 | Condition on occupancy permit Provides that a maintenance of an ESM (whether in an occupancy permit or applied through a maintenance determination) are to be treated as a condition of the occupancy permit. |  |  |  |  | ✓ | This Regulation strengthens enforcement action for regulators. Clarifies that compliance with ESM maintenance on an occupancy permit is a condition of occupying a building or place of public entertainment. Noncompliance should be enforced through s 40 of the Act. This Regulation also provides that compliance with a maintenance determination is to be treated as a condition on an occupancy permit so it can be enforced consistently. |
| 1205A | 194 | Compliance with AS 1851—2012 taken to be compliance with earlier standards Enables maintenance of ESMs to be in accordance with the AS 1851—2005 Maintenance of fire protection systems and equipment, despite what is specified in the occupancy permit condition. |  | ✓ |  |  |  | This Regulation reduces burden and improves safety by allowing for owners to maintain the ESMs in accordance with another standard, which is considered to be less onerous to comply with than earlier versions of that standard. |

#### Div 4—Display and inspection requirements for occupancy permits

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1007 | 195 | Display of occupancy permit at approved location Requires the owner to ensure a copy of the occupancy permit to be displayed in an approved location. |  |  |  |  | ✓ | Addresses information asymmetry for building occupants. Facilitates compliance, monitoring and enforcement. This is now an infringement offence. It is now straight forward to enforce because the words 'take all reasonable steps' are replaced with 'must ensure'. |
| 1008 | 196 | Approved location for display of permit Specifies that the approved location to display a copy of the occupancy permit must be in a prominent position accessible to the occupants. |  | ✓ |  |  |  | This Regulation enables people who occupy a building of a public nature or PoPE to view and comply with the occupancy permit. |
| 1009 | 197 | Owner to make permit available for inspection Requires the owner to ensure a copy of the occupancy permit is available for inspection at the building or PoPE on request by the MBS or chief officer. |  |  |  |  | ✓ | This Regulation facilitates efficiency by ensuring that the information required to monitor compliance is always readily available. This requirement is drafted as a strict liability offence so determination of culpability is straight forward. This is now an infringement offence. It is now straight forward to enforce because the words 'take all reasonable steps' are replaced with 'must ensure'. |

#### Div 5—Certificates of final inspection

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1006 | 198 | Form of certificate of final inspection Requires the certificate of final inspection to be issued in the prescribed form. |  |  | ✓ |  |  | This Regulation is necessary because a certificate of final inspection is used to calculate certain time periods in the Act and the Regulations (where an occupancy permit is not required) for liability, insurance and other matters. Additional information required to be contained in the certificate of final inspection in relation to any performance solutions used or if it was subject to a combined allotment decision (see proposed form 19). |
| 1015 | 199 | Building surveyor to notify chief officer of issue of certificates of final inspection Requires the RBS to notify a chief officer within 10 days of issuing a certificate of final inspection involving of an alarm to be connected to a fire station or other approved monitoring service, or where they are the reporting authority. |  | ✓ |  |  |  | This Regulation mitigates health and safety risks by ensuring that the reporting authority has been able to check how the recommendations were implemented. |

#### Div 6—Records

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1010 | 200 | Keeping of occupancy permit records Requires council to retain documents relating to occupancy permits. |  |  |  | ✓ |  | This Regulation has been amended to no longer required to keep documents in their ‘original form’. Aligns requirements with the role of council under the Public Records Act 1973. |
| 1012 | 201 | Documents to be given to council Requires the RBS to give council a copy of the certificate of final inspection and certificates of inspection issued for mandatory notification stages. |  |  | ✓ |  |  | This Regulation addresses information asymmetry and facilitates monitoring and enforcement. The details that must be provided to the council are now specified. |
| 1013 | 202 | Prescribed time to give council other documents Specifies the documents that a RBS must give to the council. |  |  | ✓ |  |  | This Regulation gives effect to s 73(1A) of the Act. Additional documents are now specified. |
| 1014 | 203 | Information to be supplied to council Requires the documents to be forwarded to council within 7 days after issuing occupancy permit or temporary approval. |  | ✓ |  |  |  | This Regulation gives effect to s 73(1A) of the Act. |

### Pt 15—Places of public entertainment

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1101 | 204 | Application Clarifies what buildings must obtain an occupancy permit to provide public entertainment. |  | ✓ |  |  |  | This Regulation specifies the buildings that must comply with pt. 15 of the proposed Regulations. |
| 1102 | 205 | Prescribed classes of buildings or places Specifies the buildings and places that constitute PoPEs. |  | ✓ |  |  |  | This Regulation sets out when a building and place must comply with requirements in the Act. There is an exemption for community based organisations. |
| 1104 | 206 | Prescribed temporary structures Establishes the types of structures that constitute 'temporary structures'. |  | ✓ |  |  |  | This Regulation sets out when a temporary structure must comply with requirements in the Act. |
| 1803 | 207 | Exemption from permit for temporary structures This Regulation exempts a temporary structure from the requirement to obtain a building permit. |  | ✓ |  |  |  | This Regulation states when a building permit is not required for temporary structures. This Regulation has been updated to modernise the drafting structure. This assists the general administration of the Act. |
| 1103 | 208 | Prescribed place of public entertainment Specifies when an occupancy permit must be issued by VBA. |  |  | ✓ |  |  | This Regulation gives effect to s 53(2) of the Act. Melbourne Cricket Ground, Yarra Park and East Melbourne have been removed. There is no change in process that is required to be followed when applying for an occupancy permit, there is only a change in who can issue the occupancy permit. |
| 1105 | 209 | Conditions on occupancy permits Specifies the types of conditions that an occupancy permit can be made subject to. |  | ✓ |  |  |  | This Regulation authorises a RBSs to apply a more expansive range of conditions due to the nature of public entertainment and the specific risks associated with it. |
| 1106 | 210 | Structural design of prescribed temporary structures Establishes the requirement for prescribed temporary structures to be structurally safe and designed in accordance with engineering principles. |  | ✓ |  |  |  | This Regulation provides for the safety of the public who use temporary structures, |

### Pt 16—Maintenance of buildings and places of public entertainment

#### Div 1—Maintenance of essential safety measures

##### Sub-div 1—General

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1201 1212 | 211 | Application of this division Specifies the buildings that must comply with pt. 12. Only private residential dwellings and nonhabitable structures are excluded. |  |  | ✓ |  |  | This Regulation specifies the buildings that must comply with pt. 16. Includes a Class 4 part of a building in buildings constructed before 1 July 1994. |
| 1202 1213 | 212 | Definitions in this division Provides definitions which only apply to pt. 16 of the proposed Regulations. |  |  | ✓ |  |  | This Regulation promotes consistent interpretation. The definition of 'essential safety measure' is wider for pt. 16 of the proposed Regulations. |

##### Sub-div 2—Maintenance determinations

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1204 | 213 | Maintenance determinations for ESMs in buildings and or PoPEs Requires the RBS must make a determination, if an ESM is impacted or required by building work that does not require an occupancy permit. |  |  | ✓ |  |  | This Regulation ensures that requirements for ESMs are still imposed, if they can not be imposed through the issue of an occupancy permit. The maintenance determination must now be in the approved form. |
| 1205 | 214 | Owner must comply with maintenance determination Specifies maintenance requirements in relation to a maintenance determinations for buildings or PoPEs. |  |  |  |  | ✓ | This Regulation mitigates health and safety risks by ensuring there is an obligation to comply with a maintenance determination when proposed Regulation 193(2) will not apply. The Regulation has been updated to apply only to a building does not have an occupancy permit. The penalty has increased from 10 to 20 penalty units because noncompliance with a maintenance determination could result in injury and death. |
| 1205A | 215 | Compliance with AS 1851—2012 taken to be compliance with earlier standards Enables maintenance of ESMs to be in accordance with the AS 1851—2005 Maintenance of fire protection systems and equipment despite what is specified in relation to the maintenance determination. |  | ✓ |  |  |  | This Regulation reduces burden and improves safety by allowing for owners to maintain the ESMs in accordance with another standard, which is considered to be less onerous to comply with than earlier versions of that standard. |

##### Sub-div 3—Maintenance schedules

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| n/a | NEW 216 | RBS must prepare or update maintenance schedule Requires a maintenance schedule to be prepared in certain circumstances for buildings with existing ESMs installed. The owner must provide all relevant records to prepare it. |  |  |  |  | ✓ | This Regulation mitigates health and safety risks from information asymmetry. A new Regulation to provide all current and future building owners with a single point of reference in relation to the ESMs in the buildings and the maintenance obligations associated with them. A penalty applies to this Regulation. |
| 1206 | Multiple | Has been replaced by two Regulations. Discussed immediately below in relation to proposed Regulations 217, 218 and 220 |  |  |  |  |  |  |
| n/a | NEW 217 | Owner of building or place may apply for maintenance schedule Enables a building surveyor to provide a service preparing maintenance schedules. |  |  |  |  | ✓ | This Regulation ensures maintenance schedules are prepared by appropriately qualified persons and in accordance with the Regulations where an owner at their own discretion elects to have one prepared. Very similar to current reg 1206. |
| 1206(1) | 218 | Building surveyor may prepare maintenance schedule Enables an owner to apply to a building surveyor for preparing a maintenance schedule. |  | ✓ |  |  |  | This Regulation reduces information asymmetry. Allows for building owners to improve their records and requirements in relation to ESMs at their own discretion. |
| n/a | NEW 219 | Building surveyor must give owner copy of maintenance schedule Requires a copy of the maintenance schedule to be provided to the owner of the building or place without delay. |  |  |  |  | ✓ | This Regulation ensures the owner has a copy when one is prepared. This requirement has been specified to reduce administrative delays. |
| 1206(2) 1206(4) | 220 | Maintenance schedules Specifies the form and information that must be contained in a maintenance schedule. |  |  |  |  | ✓ | This Regulation requires all ESMs in a building to be listed in a maintenance schedule with the corresponding maintenance requirements. It must be in the form approved by VBA to facilitate consistent record-keeping across all buildings. |

##### Sub-div 4—Reporting and records

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1208 1214 | 221 | Owner must prepare annual ESMs report Requires an owner of a building or PoPE to prepare a report annually. |  |  | ✓ |  |  | This Regulation mitigates the risk of ESMs deteriorating by requiring an owner to report on maintenance. Has been updated to amalgamate two Regulations. The penalty has increased from 10 to 20 penalty units to indicate the importance of this requirements, as it is an important record for undertaking monitoring and enforcement activity. |
| 1209 1215 | 222 | Contents and form of annual ESMs report Specifies the content and form of an annual report. |  |  |  |  | ✓ | To facilitate monitoring and enforcement and ensure that all relevant information is provided to assess compliance and improve record-keeping. Must be in the form approved by VBA to facilitate complete and consistent reporting across all buildings. |
| 1207 1216 | 223 | Records relating to ESMs must be made available Specifies the ESMs records that an owner must have available for inspection relating to a building or place of public entertainment. |  |  | ✓ |  |  | This Regulation facilitates monitoring and enforcement and ensure that all relevant information is provided to assess compliance and improve record-keeping. Has been updated to amalgamate two Regulations. The penalty has also increased from 10 to 20 penalty units to indicate the importance of these records being available for monitoring and enforcement purposes, and it also encourages good record-keeping. |
| 1217(1) | 224 | Maintenance responsibility of owner of building or place constructed before 1 July 1994 Specifies a general obligation to maintain an ESM that was installed before 1 July 1994. |  |  |  |  | ✓ | This Regulation ensures that ESMs installed in a building are maintained if there is no specific corresponding maintenance requirement imposed through an occupancy permit or maintenance determination. The penalty has increased from 10 to 20 penalty units to indicate the seriousness of this breach. If an ESMs fails the impacts can be very high. |
| 1217(2) | 225 | ESM not to be moved from approved locations This Regulation makes it an offence to move an ESM for any purpose other than maintenance, inspection or testing. |  |  |  |  | ✓ | This Regulation mitigates health and safety risks that could arise if ESMs are tampered with. The offence now applies to all buildings and the penalty has increased from 10 to 20 penalty units. |

#### Div 2—Maintenance of exits and paths of travel relating to buildings or places of public entertainment

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1218 | 226 | Maintenance of exits by occupiers of buildings or PoPEs Requires all occupants of a building to maintain paths of travel and points of egress. |  |  | ✓ |  |  | This Regulation mitigates health and safety risks by ensuring that points of egress are not compromised by occupant behaviour, such as chaining doors shut or using hallways or fire extinguishers cupboards for storage space. These actions can severely impact on the ability to evacuate a building during an emergency. This Regulation has been updated to remove the word ‘maintain’ to clarify that maintenance obligations lie with owners and cannot be passed onto occupiers. This change is consistent with owners responsibilities under s 52 of the Retail Leases Act 2003 and s 68 of the Residential Tenancies Act 1997, as well as a VCAT advisory opinion of Justice Garde on 1 May 2015. |

### Pt 17—Existing buildings

#### Div 1—Change of use of existing buildings

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1011(1) 1011(5) | 227 | Change of use This Regulation prohibits a person from changing the use of a building unless there is an occupancy permit allowing that use and the building complies with the Regulations applicable to that use. |  |  |  |  | ✓ | This Regulation protects health and safety by ensuring that buildings have the appropriate safety features and services connected. The exemption to this Regulation has been moved to pt. 18. The penalty has been increased and introduces a prescribed form to facilitate clear and consistent records of these decisions across all building surveyors. |
| 1011(4) | 228 | PBS may exercise powers of MBS under reg 227 Enables a PBS who has been appointed to issue an occupancy permit may exercise the powers of a MBS to grant an exemption from compliance with any of the Regulations applicable to the new use. |  | ✓ |  |  |  | This Regulation creates efficiencies in the delivery of building services. The matters that must be considered when deciding whether to grant an exemption from compliance are consistent with those matters that need to be considered when issuing an occupancy permit. |

#### Div 2—Subdivision of existing building

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 503 | 229 | Subdivision of existing building Requires each building resulting from a subdivided building to be brought into compliance with current Regulations. |  |  |  |  | ✓ | This Regulation protects health and safety by ensuring that existing buildings are upgraded to modern day standards when subdivided. Penalty has increased and introduces a prescribed form to facilitate clear and consistent records of these decisions across all building surveyors. |
| 503(4) | 230 | PBS may exercise powers of MBS under reg 1303 Enables a PBS who has been appointed to issue an occupancy permit to exercise the powers of a MBS to exempt a subdivided building from being brought into conformity with the Regulations. |  | ✓ |  |  |  | This Regulation creates efficiencies in the delivery of building services. The matters that must be considered when deciding whether to grant an exemption are consistent with those that need to be considered when issuing an occupancy permit. |

#### Div 3—Alterations

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 608(2) 608(3) | 231 | Alteration to existing building Requires any alteration to an existing building to comply with current Regulations and specifies when the entire building will be required to be upgraded. Also provides for the RBS to consent to partial compliance for alterations. |  |  |  |  | ✓ | This Regulation protects health and safety by ensuring that existing buildings are upgraded to modern day standards when altered. Introduces a prescribed form to facilitate clear and consistent records of these decisions across all building surveyors. |
| 609(1) | 232 | Alterations affecting exits and paths to exits Requires compliance with Section D of Volume One of the BCA if an alteration would adversely affect an exit or path of travel. |  |  |  |  | ✓ | This Regulation protects health and safety by providing for suitable exits and paths to exits. Introduces a prescribed form to facilitate clear and consistent records of these decisions across all building surveyors. |
| 611 | 233 | Extension of noncomplying external wall This Regulation exempts a short noncomplying external wall of the type from having to comply with P2.3.1 of Volume Two of the BCA. |  | ✓ |  |  |  | This Regulation reduces unnecessary costs associated with low-risk construction (extension of an external wall). |
| 116 | 234 | Application of requirements in the BCA Volume One relating to access to buildings for persons with disabilities This Regulation modifies operation of the BCA in certain circumstances to align the BCA Volume One with the Disability (Access to Premises-Buildings) Standards 2010 of the Commonwealth in relation to new parts of buildings. |  | ✓ |  |  |  | This Regulation ensures that people who require special access to use buildings and PoPEs are accommodated and ensures future development will comply with Commonwealth rules in relation to accessibility standards. |

#### Div 4—General

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 122 | 235 | Small live-music venue classification This Regulation modifies the BCA by excluding a small live-music venue from being classified as a Class 9b. |  | ✓ |  |  |  | To reduce unnecessary costs of upgrading buildings faced by small cafes, restaurants, pubs, bars, and other Class 6 venues seeking to host live music. |

### Pt 18—Building product accreditation

#### Div 1—Definitions

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1401 | 236 | Definitions Provides for definitions for 'building product' and 'committee'. |  | ✓ |  |  |  | This Regulation aligns with ss 14 and 15 of the Act, to assist in the general administration of the Act. Promotes consistency in interpretation. |

#### Div 2—Accreditation persons and bodies

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1402 | 237 | Prescribed persons and bodies to accredit building products Provides that for the purposes of accreditation, the Australian Building Codes Board and a person or body authorised by the Australian Building Codes Board are prescribed. |  |  | ✓ |  |  | This Regulation has been updated to include a person or body authorised by the Australian Building Codes Board as a prescribed person or body for the purposes of ss 14 and 15 of the Act. This ensures that the Regulation is in line with the CodeMark accreditation scheme. |
| 110 | 238 | Definition of Certificate of Accreditation in the BCA Provides for a variation to the BCA to support the definition of 'Certificate of Accreditation'. |  |  | ✓ |  |  | This Regulation has been redrafted to support the definition of 'Certificate of Accreditation' in BCA to provide reference to BRAC. |

#### Div 3—Accreditation by Committee

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1403 | 239 | Application of division Provides that pt 14 div 3 does not apply to building products that comply with the relevant deemed-to-satisfy provisions of the BCA. |  | ✓ |  |  |  | This Regulation ensures consistency with the BCA. |
| 1404 | 240 | Application for accreditation Provides for applications to BRAC to have a building product accredited. |  |  |  |  | ✓ | This Regulation has been strengthened to provide that applications must be in a required form, to provide for administrative efficiency in the accreditation process. |
| 1410 | 241 | Information in application to remain confidential Specifies an offence of 10 penalty units to ensure that information provided as part of an application for accreditation will remain confidential. |  | ✓ |  |  |  | Current reg 1410 renumbered. This Regulation protects an applicant for accreditation’s privacy. |
| 1405 | 242 | Accreditation fees Provides the fee for accreditation of a building product, to be paid to the Victorian Building Authority Fund. |  | ✓ |  |  |  | This Regulation prescribes the application fee. The fee continues to be 138.31 fee units, to assist in the administration of accreditation of building products under the Act. |
| 1406 | 243 | Certificate of accreditation Specifies that if a building product receives accreditation by BRAC, a certificate of accreditation is to be issued. |  |  | ✓ |  |  | This Regulation promotes consistency with the national accreditation scheme (CodeMark) and to provide additional transparency in the administration of accreditation certificates, the Regulation now lists the information that must be set out in the certificate. |
| 1408 | 244 | Revocation of accreditation Specifies that the circumstances in which accreditation of a building product can be revoked by the Building Regulations Advisory Committee. |  | ✓ |  |  |  | This Regulation provides both consumer and general protections to the building industry, this Regulation continues to provide that the accreditation of building products can be revoked in some circumstances, including fraud and unsatisfactory standards. |
| n/a | NEW 245 | Holder may request revocation of accreditation Establishes a mechanism whereby a certificate of accreditation holder may request the Building Regulations Advisory Committee to revoke an application. |  |  |  |  | ✓ | This Regulation provides flexibility in the event that a building product is proposed to cease manufacturing or a certificate of accreditation holder discovers it is no longer compliant (for example, a change in standard). This assists in maintaining the integrity of the accreditation process. |
| n/a | NEW 246 | Accreditation ceases to have effect on revocation Clarifies that accreditation ceases to have effect on revocation. |  |  |  |  | ✓ | This Regulation assists in the administration of the accreditation scheme, this Regulation provides that accreditation ceases to have effect when revoked. |
| 1411 | 247 | Offence to falsely claim product accredited Provides for an offence for a person or body falsely claiming that a building product is accredited. |  | ✓ |  |  |  | This Regulation maintains an offence of 20 penalty units for falsely claiming that a building product is accredited. This provides a deterrent to false accreditations in the interests of building safety and consumer protection. |

#### Div 4—Register and records

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1407 | 248 | Register of accredited products Specifies that VBA must keep a register of all building products which have been accredited. |  |  | ✓ |  |  | This Regulation assists in the administration of the register kept by VBA, this Regulation has been updated to specify the information that the register must contain. In the interests of increasing the availability of information to consumers, the Regulation also clarifies that access to the register is available to the public. |
| 1407(b)(i) | 249 | Notice of accreditation Provides an obligation on VBA to publish notice of building product accreditation in the Victorian Government Gazette. |  |  | ✓ |  |  | This Regulation has been updated to contain the existing obligation for VBA to publish building product accreditations. It has been relocated to a separate Regulation for ease of reference. Updating the requirement to prescribe the information that must be published provides transparent information to consumers and consistency with the national accreditation framework. |
| 1407(b)(ii) | 250 | Notice of revocation of accreditation Provides an obligation on VBA to publish notice of building product accreditations that have been revoked in the Victorian Government Gazette. |  |  | ✓ |  |  | The existing obligation for VBA to publish building product accreditations that have been revoked has been relocated to a separate Regulation for ease of reference. Updating the requirement to prescribe the information that must be published ensures transparency of process and information to consumers. |
| 1409 | 251 | Records Provides record-keeping requirements of VBA in relation to the accreditation of building products. |  |  |  |  | ✓ | This Regulation has been strengthened to provide that VBA must retain copies of all certificates of accreditation issued by the Building Regulations Advisory Committee, to assist administrative processes. |

### Pt 19—Building practitioners

#### Div 1—Appointment of private building surveyor

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1504 | 252 | Period for which PBS may not act Specifies the period for which a PBS may not act. |  | ✓ |  |  |  | This Regulation continues to prescribe a period of 1 year to give effect to s 79(1)(b) of the Act, assisting in preventing PBS conflicts of interest. |

#### Div 2—Registration

##### Sub-div 1—Initial application for registration

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| n/a | NEW 253 | Applications for registration Establishes the power for approved forms for applications for registration. |  |  |  |  | ✓ | This Regulation codifies VBA’s current registration processes and assists in providing transparency of process for applicants seeking registration as a building practitioner. |
| n/a | NEW 254 | Information to accompany application for registration Specifies the information to accompany an application for registration. |  |  |  |  | ✓ | This Regulation codifies VBA’s current registration processes and provides transparency of process for applicants seeking registration as a building practitioner. |
| 1509 | 255 | Prescribed information as to character of applicant for registration Specifies the information as to character to be provided by an application for registration. |  | ✓ |  |  |  | This Regulation continues to prescribe information to assist in the determination of an applicant’s character to give effect to s 169(2)(ca) of the Act. This assists in providing a robust assessment of applicants to protect consumers. |

##### Sub-div 2—Prescribed qualifications and classes of building practitioner

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1503 | 256 | Prescribed qualifications and classes of building practitioner Defines prescribed qualifications for the purposes of registration. |  |  | ✓ |  |  | This Regulation gives effect to s 170(1)(b)(i) of the Act by defining prescribed qualifications and practical experience. This provides consumers and applicants with transparent parameters about registration as a building practitioner. The definition of 'practical experience' is new and prescribed qualifications has been included to be consistent with s 175 of the Act. |

##### Sub-div 3—Condition on registration

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1512 | 257 | Condition of registration Establishes that a building practitioner must not allow another person to use their certificate as condition on registration. |  | ✓ |  |  |  | This Regulation gives effect to s 170A of the Act. The condition on registration ensures that a registered building practitioner does not facilitate other people claiming to be registered when they are not. |

#### Div 3—Application for renewal of registration

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| n/a | NEW 258 | Applications for renewal of registration Establishes approved forms for applications for renewal of registration. |  |  |  |  | ✓ | This Regulation codifies VBA’s current registration processes by establishing the power for an approved form for renewal of registration. This assists in providing transparency of process for applicants seeking registration as a building practitioner. |
| n/a | NEW 259 | Renewal criteria for application for renewal of registration Specifies the renewal criteria to accompany an application for registration. |  |  |  |  | ✓ | This Regulation prescribes evidence of proof of identity as the accompanying information to be provided for renewal of registration, to supplement s 172AA of the Act. This provides transparency of process for applicants seeking registration as a building practitioner. |

#### Div 4—Disciplinary proceedings and action for building practitioners

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1511 | 260 | Notice of suspension or cancellation of registration Establishes forms to be provided for notice of suspension or notice of cancellation of registration. |  | ✓ |  |  |  | This Regulation assists in administrative efficiency for building practitioners by providing template forms for suspension of registration or notice of cancellation of registration. |

#### Div 5—Professional standards

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1502 | 261 | Professional standards Specifies the professional standards to which a building practitioner must abide. |  | ✓ |  |  |  | The Regulation continues to specify the professional standards to be observed by registered building practitioners in the different categories or classes of registration. This supplements VBA’s registration assessment process and provides standards by which to base disciplinary action. |

#### Div 6—Fees

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1508 | 262 | Duplicate certificate fee Prescribes a fee for a duplicate practitioners certificate. |  | ✓ |  |  |  | The Regulation gives effect to s 172(7) of the Act, to prescribe a fee of 2.75 fee units for a duplicate building practitioner’s certificate. This assists the general administration of the Act. |

#### Div 7—Exemptions for certain building practitioners

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1810 | 263 | Builders engaged in low value domestic building work Specifies that persons engaged in low value domestic building work are exempt from registration as a building practitioner. |  |  | ✓ |  |  | This Regulation has been updated for consistency with the Domestic Building Contracts Act 1995, to allow those engaging in low value domestic building work to be titled and considered a building practitioner. This assists the general administration of the Act. |
| 1811 1812 | 264 | Builders of Class 10 buildings Specifies that builders engaged in the building of Class 10 and Class 10b buildings are exempt from registration as a building practitioner. |  |  | ✓ |  |  | This Regulation has been updated for consistency with the BCA and to consolidate the exemptions relating to Class 10 and Class 10b buildings. This assists the general administration of the Act. |
| 1813 | 265 | Draftsperson doing certain domestic building work Specifies that draftspersons engaged in preparing plans and documentation are exempt from registration as a building practitioner. |  |  | ✓ |  |  | This Regulation has been updated for consistency with the Domestic Building Contracts Act 1995, to allow draftspersons engaging in the preparation plans and related documentation to be titled and considered a building practitioner. This assists the general administration of the Act. |

### Pt 20—Appeals and reviews

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1601 | 266 | Appeal periods Prescribes time periods for decisions of the BAB after an appeal is made. |  |  | ✓ |  |  | This Regulation has been updated to clarify the sections of the Act under which right of appeal arises, the subject matter of the decisions relating to the right of appeal and the prescribed appeal period for the purposes of s 146(1)(a) of the Act. This provides additional clarity for those appealing and awaiting the effect of a BAB decision. |
| 1602 | 267 | Appeal, referral and application fees Prescribes fees for making an appeal, a referral or an application to BAB. |  | ✓ |  |  |  | This Regulation assists the general administration of the appeals process and enables BAB to recover costs for services. The structure of the Regulation has been updated into three subregulations. All these fees are set out in detail in Chapter B4.2.4. The fee units for appeals range from 13.82 and 23.04 for Class 1 & 10 buildings to 23.04 and 46.10 for Class 2–9 buildings. There is also a fast-track fee of 36.88 fee units. The fee units for modification and compliance range from 8.29 fee units for Class 1 & 10 buildings to 18.43 fee units for Class 2–9 buildings. Referrals (design) cost 18.43 fee units and referrals (access provisions) is 41.89 fee units. |
| 1602(f), 1603, 1604 | 268 | Fees for fast-track appeals Prescribes fees for fast-track appeals, hourly based fees and the refund of a request fee if the request fee is refused. |  |  | ✓ |  |  | This Regulation consolidates and updates existing Regulations for clarity and to assist the administration of the appeals framework. A fee for site inspections has been removed, however, as the department understands that this activity is only undertaken for fast-track appeals, proposed fee 268(2) is considered sufficient. The fee for making a request is 36.88 fee units, and an additional prescribed fee is 14.74 fee units an hour. |
| 1604(2) | 269 | VBA must refund fees Provides that VBA may refund fees paid to BAB for an appeal, request, application or referral that does not proceed. |  | ✓ |  |  |  | This Regulation maintains current reg 1604(2) and allows refunds to appeal proceedings that do not eventuate. This ensures that appeal applicants do not incur unnecessary expense. |
| 1605 | 270 | Review period Provides a period of 60 days to lodge a review with the Victorian Civil and Administrative Tribunal. |  | ✓ |  |  |  | This Regulation continues to prescribe a 60 day period for reviews to the Victorian Civil and Administrative Tribunal. |

### Pt 21—Infringement notices

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1701 | 271 | Application Pt 17 applies to all buildings and PoPEs, whenever constructed. |  | ✓ |  |  |  | Specifies the buildings that can be subject to an infringement offence under pt. 21. |
| 1702 | 272 | Authorised officers Specifies the prescribed office or class of officers to give effect to the definition of 'authorised officer' for building infringement notices. |  |  |  |  | ✓ | This Regulation is required continues to give effect to s 254 of the Act. It has been updated to provide the offences for which authorised persons are best placed to have responsibility for enforcing. |
| 1703 | 273 | Prescribed offences Specifies the offences in the Regulations to give effect to the definition of 'prescribed offences' for building infringement notices. |  |  |  |  | ✓ | This Regulation required to prescribe offences for the purposes of s 254 of the Act, which assists in the administration of the building infringement notice framework. Infringement offences relation to fire safety in certain residential buildings have been removed (current regs 707–710), as noncompliance with these requirements needs to be treated seriously through a building notice or order. Has also been updated to include proposed reg 49(1) (current reg 317(1)) and proposed reg 195(2) (current reg 1007(2)), and align with proposed changes to pt 16 div 1 (maintenance of ESMs). |
| 1705 | 274 | Prescribed penalties Specifies the prescribed penalties for building infringement notices. |  | ✓ |  |  |  | This Regulation continues to prescribe five penalty units in the case of a PoPE and two penalty units in any other case, to give effect to s 255(3) of the Act. This assists in the administration of the building infringement notice framework. |

### Pt 22—Exemptions

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 1802 | 275 | Exemptions from building Regulations Establishes that the buildings and building work specified in sch 3 are exempt from the building Regulations. |  |  | ✓ |  |  | This Regulation has been updated to modernise the drafting structure. Item 1: Exemption for construction of a 10 m2 shed has been updated to provide for the construction of a shed on otherwise undeveloped land. Item 12: Exemption for construction of a sign has been clarified so that the exemption applies to a Class 10b structure constructed to display a sign. |
| 1803 | 207 | Exemption from permit for temporary structures. Moved to pt. 15— Places of public entertainment |  |  |  |  |  |  |
| 1804 | 276 | Exemption for Class 10 buildings on farm land Establishes that a relevant council may exempt Class 10 buildings on farm land for the purposes of the Regulations. |  | ✓ |  |  |  | This Regulation continues to provide that a relevant council may exempt Class 10 buildings on farm land for the purposes of the Regulations. |
| 1805 | 277 | Exemptions relating to building surveyors for certain buildings Provides exemptions from certain notice provisions under the Act in relation to certain public buildings. |  |  | ✓ |  |  | This Regulation has been updated to reflect current legislation. |
| 1807 | n/a | Definitions Specifies definitions for 'exemptions' relating to builders of multistorey residential buildings. | ✓ |  |  |  |  | This Regulation is proposed for revocation as the Act (specifically a domestic building insurance ministerial order made under s 135) provides for the same exemption. |
| 1808 | n/a | Exemption from required insurance for builder Establishes an exemption from insurance for a builder who proposes to or carries out domestic building work under a major domestic building contract for the construction of a multistorey residential building. | ✓ |  |  |  |  | This Regulation is proposed for revocation as the Act (specifically a domestic building insurance ministerial order made under s 135) provides for the same exemption. |
| 1809 | n/a | Exemptions from required insurance for owner-builders Establishes an exemption from insurance for an owner-builder who constructs a multistorey residential building. | ✓ |  |  |  |  | This Regulation is proposed for revocation as the Act (specifically a domestic building insurance ministerial order made under s 135) provides for the same exemption. |

### Pt 23—Transitional provisions

| 2006 Regulation | 2017 Regulation | Short description of the Regulation | Proposed regulatory change |  |  |  |  | Why the 2017 Regulation is proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Revoke | No Change | Updated | Reduce | Strengthen |  |
| 2002 | 278 | Registration of building practitioners not affected by revocation of Building (Interim) Regulations 2005 This is a transitional provision for building practitioners registered as part of the Building (Interim) Regulations 2005. |  | ✓ |  |  |  | The Regulation has been retained to ensure that building practitioners previously registered are not affected by updating of the Regulations. |
| 2003 | n/a | Transitional provisions—Building Amendment Regulations 2009 Provides for building practitioners registered using the qualifications listed in sch 7 before the commencement of the Building Amendment Regulation 2009. | ✓ |  |  |  |  | This Regulation is now redundant and will be revoked. |
| 2004 | 279 | Transitional provision—Building Legislation Amendment (Consumer Protection) Act 2016 Provides that the amendments as a result of s 38 of the Building Amendment (Consumer Protection) Act 2016 (a builder must not appoint a PBS on behalf of an owner for whom domestic building work is to be carried out; and a PBS must not accept such an appointment) do not apply in relation to a contract for domestic building work entered into before the commencement of that section. |  | ✓ |  |  |  | This Regulation has been retained to ensure that contracts entered into before the commencement of s 38 of the Building Amendment (Consumer Protection) Act 2016 are not affected. |
| n/a | NEW 280 | Transitional provision – revocation of Regulations prescribing fees This Regulation will revoke all fee Regulations by 1 June 2020. |  |  |  | ✓ |  | This Regulation gives additional time for VBA and BAB to undertake a review of all fee Regulations. It also enables a review of the proposed fees of council within three years. New fee Regulations will need to be made before June 2020 otherwise there will be no power for council, VBA or the BAB to charge these fees after that date. |

delwp.vic.gov.au

The Building Practitioners Board was abolished on 1 September 2016. From that date VBA has assumed responsibility for functions previously the responsibility of the Building Practitioners Board.

Victorian Parliamentary Debates, Legislative Assembly, vol. no. 414, Thursday, 11 November 1993, page no. 1690.

Building Regulations 2006, reg 109.

The meaning of ‘relevant building surveyor’ is defined in s 3(1) of the Building Act 1993.

Building Act 1993, pt 3 and sch 2.

Building Act 1993, s 17.

Building Act 1993, ss 19, 24 and 24A.

Building Act 1993, pt 3 and sch 2.

Building Act 1993, pt 4. Building Regulations 2006, pt 9.

Building Act 1993, div 2 of pt 4.

Building Act 1993, div 3 of pt 4, and pt 5. Building Regulations 2006, pt 10.

VBA’s 2015–16 Annual Report, available at: http://www.vba.vic.gov.au/__data/assets/pdf_file/0006/49704/VBA-Annual-Report-2015-16.pdf

Building Act 1993, div 3 of pt 12.

Building Act 1993, pt 10.

Building Act 1993, div 4 of pt 12.

Building Act 1993, S 212 and div 5 of pt 12.

Building Act 1993, s 80.

Building Act 1993, s 3(1).

Building Act 1993, s 17 and s 76(a) for PBSs.

Building Act 1993, s 76(b) for PBSs.

Building Act 1993, s 76(c) for PBSs.

Building Regulations 2006, sch 7.

Victorian Government Printer, (2005), Building Practitioners' Insurance Ministerial Order, Victoria Government Gazette. No. S 91, Thursday 12 May 2005,

Building Act 1993, s 34.

http://planningschemes.dpcd.vic.gov.au.

A planning permit may be required to change the use of land, or to construct a building or alter existing buildings on certain land. For example to open a bar, subdivide land, build two or more dwellings, construct one dwelling on a small allotment, or to undertake building work when the zone or overlay requires a permit.

Building Act 1993, pt 11.

www.accc.gov.au

https://www.vmia.vic.gov.au/insure/domestic-building-insurance

Information on domestic building insurance can be found at: www.dbi.vmia.vic.gov.au.

BCA accepted evidence of suitability for use

Pt 4 of the Residential Tenancies Act 1997 and Residential Tenancies (Rooming House Standards) Regulations 2012. Prescribed accommodation comprises: residential accommodation, hotels and motels, hostels, student dormitories, holiday camps, rooming houses.

Public Health and Wellbeing Regulations 2009

Residential Tenancies (Rooming House Standards) Regulations 2012

Commonwealth, Parliamentary Debates, Senate, 28 February 2013, 1323 (Kate Lundy) 'Building Materials Imports' http://parlinfo.aph.gov.au/parlInfo/search/display/display.w3p;query=Id%3A%22chamber%2Fhansards%2Fbbf08272-5de4-436c-9325-d389def0cc1c%2F0166%22

Occupational Health and Safety Regulations 2007

Environment Protection (Residential Noise) Regulations 2008

Victorian Competition and Efficiency Commission 2005, Housing regulation in Victoria: Building better outcomes.

Government of Victoria, 2014, Victorian Guide to Regulation, Department of Treasury and Finance, Melbourne.

Victorian Auditor General's Office 2015, Victoria’s Consumer Protection Framework for Building Construction.

The survey showed that the building industry ranked fourth out of 22 sectors, behind telecommunications (ranked 1) and above utilities (ranked 6).

Mills, A, Love, P and Williams, P (2009) Defect Costs in Residential Construction, Journal of Construction Engineering and Management, citing various previous studies.

We are not aware of any similar studies conducted on more recent data examining defect costs in detail. 
The Australian Consumer Survey 2016 found that 17% of survey respondents (Australia wide) reported experiencing a problem with a builder. This indicates building work quality remains an issue in the industry (The Australian Treasury on behalf of Consumer Affairs Australia and New Zealand (CAANZ), Australian Consumer Survey 2016, 18 May 2016, available at: http://consumerlaw.gov.au/australian-consumer-survey )

Ibid.

Ibid.

Ibid.

Victorian Building Commission (2005) Dispute reduction research summary report, March 2005

Ibid.

VBA’s building complaints system doesn’t have a categorisation process. The sample of complaints were categorised according to what appeared to be the most prominent complaint at the time of categorisation, based on the available information.

Productivity Commission 2011, Identifying and Evaluating Regulation Reforms, Research Report, Canberra. Page XVIII.

Victorian Guide to Regulation, Toolkit 3: Requirements and processes for making subordinate legislation, page 18 (Department of Treasury and Finance, updated December 2014).

ALP, Keeping it Liveable. Labor’s plan for your community (November 2014), page 3.

Building Amendment (Construction of Swimming Pools and Spas) Regulations 2016 S.R. No. 104/2016

Building Amendment Regulations 2016 S.R. No. 33 / 2016

Building Amendment (Siting Requirements) Regulations 2016 S.R. No 49/2016

S 16 of the Building Act 1993.

Net present value.

There is a Guidelines currently in place and is available on VBA’s website: http://www.vba.vic.gov.au/__data/assets/pdf_file/0006/18789/Ministers-Guidelines-MG12.pdf